ISN 3132 Wide Area Networks

NAME: DAMEN NJATENG STEVE ALANE

MATRICULE: ICTU20234425

INSTRUCTOR : DANIEL MOUNE

EXERCISE 1

Question 1.

To move from the linear to a triangular topology the following modification was done:

- The link structure was modified from n0↔n1↔n2 to  n0↔n1 , n0↔n2 ,n1↔n2

- Each node connects to one other node  that is , n0 connect only to n1  ,n1 connects to both n0 and n2  and n2  only connects to n1

- The single point of failure was removed by introducing redundancy

- Subnet moved from 10.1.1.0/24, 10.1.2.0/24 to +10.1.3.0/24 for Branch-DC link

C++ code snippet :

1. NodeContainer nodes;

2. nodes.Create(3); // n0=HQ, n1=Branch, n2=DC

3.

4. PointToPointHelper p2p;

5. p2p.SetDeviceAttribute("DataRate", StringValue("5Mbps"));

6. p2p.SetChannelAttribute("Delay", StringValue("2ms"));

7.

8. NetDeviceContainer devices_hq_branch = p2p.Install(nodes.Get(0), nodes.Get(1));

9. NetDeviceContainer devices_hq_dc = p2p.Install(nodes.Get(0), nodes.Get(2));

10. NetDeviceContainer devices_branch_dc = p2p.Install(nodes.Get(1), nodes.Get(2));

11.

Question 2

Complete Routing Logic Analysis:

a) Primary path from HQ to DC is direct:

- HQ sends to 10.1.2.2 (DC) → Matches directly connected 10.1.2.0/24

- Uses Interface 2 (HQ-DC link)

- Latency: ~2ms

b) Backup path from HQ to DC goes through Branch:

- If HQ-DC link fails, HQ needs alternative path to DC

- BUT: Static routing doesn't automatically failover

- Backup would require: Dynamic routing OR manual route change after failure

- Potential backup route: 10.1.2.0/24 → 10.1.1.2 (Branch) via Interface 1

  - Would need metric/priority to be secondary route

c) Symmetric routing for return traffic:

DC→HQ traffic: Direct via 10.1.2.0/24 (matches primary path)
DC→Branch traffic: Via HQ (10.1.1.0/24 → 10.1.2.1)
Branch→DC traffic: Direct via 10.1.3.0/24
Branch→HQ traffic: Direct via 10.1.1.0/24

Question 3

- C++ code snippet

1. // Schedule link failure at t=4 seconds

2. Simulator::Schedule(Seconds(4.0), [&]() {

3.     std::cout << "\n[Time: " << Simulator::Now().GetSeconds()

4.               << "s] PRIMARY LINK FAILURE: HQ-DC link disabled!\n";

5.

6.     // Get IPv4 objects

7.     Ptr<Ipv4> ipv4HQ = n0->GetObject<Ipv4>();

8.     Ptr<Ipv4> ipv4DC = n2->GetObject<Ipv4>();

9.

10.     // Bring down HQ's interface to DC

11.     uint32_t ifIndexHQ = ipv4HQ->GetInterfaceForAddress(Ipv4Address("10.1.2.1"));

12.     ipv4HQ->SetDown(ifIndexHQ);

13.

14.     // Bring down DC's interface to HQ

15.     uint32_t ifIndexDC = ipv4DC->GetInterfaceForAddress(Ipv4Address("10.1.2.2"));

16.     ipv4DC->SetDown(ifIndexDC);

17.

18.     std::cout << "Traffic between HQ and DC must now use backup path via Branch...\n";

19. });

20.

- C++ code snippet

1. #include "ns3/flow-monitor-module.h"

2.

3. // Install FlowMonitor on all nodes

4. FlowMonitorHelper flowmon;

5. Ptr<FlowMonitor> monitor = flowmon.InstallAll();

6.

7. // ... (simulation runs) ...

8.

9. // Analyze results after simulation

10. monitor->CheckForLostPackets();

11. Ptr<Ipv4FlowClassifier> classifier = DynamicCast<Ipv4FlowClassifier>(flowmon.GetClassifier());

12. std::map<FlowId, FlowMonitor::FlowStats> stats = monitor->GetFlowStats();

13.

14. for (auto &flow : stats) {

15.     Ipv4FlowClassifier::FiveTuple t = classifier->FindFlow(flow.first);

16.

17.     std::cout << "\nFlow " << flow.first << " (" << t.sourceAddress << " → "

18.               << t.destinationAddress << "):\n";

19.     std::cout << "  Tx Packets: " << flow.second.txPackets << "\n";

20.     std::cout << "  Rx Packets: " << flow.second.rxPackets << "\n";

21.     std::cout << "  Packet Loss Ratio: "

22.               << ((flow.second.txPackets - flow.second.rxPackets) * 100.0 / flow.second.txPackets)

23.               << "%\n";

24.

25.     if (flow.second.rxPackets > 0) {

26.         std::cout << "  Mean Delay: " << flow.second.delaySum.GetSeconds() / flow.second.rxPackets

27.                   << "s\n";

28.         std::cout << "  Mean Jitter: " << flow.second.jitterSum.GetSeconds() / flow.second.rxPackets

29.                   << "s\n";

30.

31.         // **CRITICAL VERIFICATION: Check if packets arrived after failure**

32.         if (t.sourceAddress == "10.1.1.1" && t.destinationAddress == "10.1.2.2") {

33.             double packetDeliveryRatio = (flow.second.rxPackets * 100.0) / flow.second.txPackets;

34.             if (packetDeliveryRatio > 95.0) {

35.                 std::cout << "  ✓ SUCCESS: HQ→DC traffic maintained after link failure!\n";

36.             } else {

37.                 std::cout << "  ✗ FAILURE: HQ→DC traffic interrupted!\n";

38.             }

39.         }

40.     }

41. }

42.

- C++ code snippet

1.                    #include "ns3/flow-monitor-module.h"

2.

3. // Install FlowMonitor on all nodes

4. FlowMonitorHelper flowmon;

5. Ptr<FlowMonitor> monitor = flowmon.InstallAll();

6.

7. // ... (simulation runs) ...

8.

9. // Analyze results after simulation

10. monitor->CheckForLostPackets();

11. Ptr<Ipv4FlowClassifier> classifier = DynamicCast<Ipv4FlowClassifier>(flowmon.GetClassifier());

12. std::map<FlowId, FlowMonitor::FlowStats> stats = monitor->GetFlowStats();

13.

14. for (auto &flow : stats) {

15.     Ipv4FlowClassifier::FiveTuple t = classifier->FindFlow(flow.first);

16.

17.     std::cout << "\nFlow " << flow.first << " (" << t.sourceAddress << " → "

18.               << t.destinationAddress << "):\n";

19.     std::cout << "  Tx Packets: " << flow.second.txPackets << "\n";

20.     std::cout << "  Rx Packets: " << flow.second.rxPackets << "\n";

21.     std::cout << "  Packet Loss Ratio: "

22.               << ((flow.second.txPackets - flow.second.rxPackets) * 100.0 / flow.second.txPackets)

23.               << "%\n";

24.

25.     if (flow.second.rxPackets > 0) {

26.         std::cout << "  Mean Delay: " << flow.second.delaySum.GetSeconds() / flow.second.rxPackets

27.                   << "s\n";

28.         std::cout << "  Mean Jitter: " << flow.second.jitterSum.GetSeconds() / flow.second.rxPackets

29.                   << "s\n";

30.

31.         // **CRITICAL VERIFICATION: Check if packets arrived after failure**

32.         if (t.sourceAddress == "10.1.1.1" && t.destinationAddress == "10.1.2.2") {

33.             double packetDeliveryRatio = (flow.second.rxPackets * 100.0) / flow.second.txPackets;

34.             if (packetDeliveryRatio > 95.0) {

35.                 std::cout << "  ✓ SUCCESS: HQ→DC traffic maintained after link failure!\n";

36.             } else {

37.                 std::cout << "  ✗ FAILURE: HQ→DC traffic interrupted!\n";

38.             }

39.         }

40.     }

41. }

42.

43.

NS 3 -IPLEMENTATION OF THE TRIANGULAR TOPOLOGY

EXERCISE  2

Question 1: Traffic Differentiation Implementation

Class 1: VoIP-like Traffic

Implementation Details:

Packet Size: 160 bytes (typical G.711 codec payload)

Inter-packet Interval: 20ms (50 packets per second)

Data Rate: ~64 kbps per flow

DSCP Marking: EF (Expedited Forwarding, value 46 or 0xb8 in TOS field)

Latency Requirement: < 150ms end-to-end (ITU-T G.114 recommendation)

Jitter Requirement: < 30ms

CPP code implementation

1. class VoipApplication : public Application

2. {

3.     // Packet size: 160 bytes (G.711 codec)

4.     uint32_t m_packetSize = 160;

5.

6.     // 20ms interval between packets

7.     Time m_interval = MilliSeconds(20);

8.

9.     // DSCP EF marking (46 << 2 = 0xb8)

10.     SocketIpTosTag tosTag;

11.     tosTag.SetTos(0xb8);

12.     packet->AddPacketTag(tosTag);

13. }

14.

Rationale:

- VoIP requires consistent low-latency delivery

- Small packet size minimizes serialization delay

- Regular 20ms intervals simulate real-time voice encoding

Class 2: FTP-like Traffic

Implementation Details:

Packet Size: 1500 bytes (Maximum Transmission Unit for Ethernet)

Inter-packet Interval: 10ms (configurable for congestion testing)

Data Rate: ~1.2 Mbps per flow (can scale with multiple flows)

DSCP Marking: BE (Best Effort, value 0)

Delivery: Best-effort, tolerant to delay and packet loss

CPP code Implementation

1. class FtpApplication : public Application

2. {

3.     // Large packets for bulk transfer

4.     uint32_t m_packetSize = 1500;

5.

6.     // Aggressive sending rate

7.     Time m_interval = MilliSeconds(10);

8.

9.     // Best-effort marking

10.     SocketIpTosTag tosTag;

11.     tosTag.SetTos(0x00);

12.     packet->AddPacketTag(tosTag);

13. }

14.

Rationale:

FTP represents bulk data transfer with no real-time constraints

Large packets maximize throughput efficiency

Best-effort marking indicates low priority

Tolerates retransmissions and delays

Packet Tagging Strategy

Dual Tagging Approach:

DSCP Tagging (SocketIpTosTag):

Standard IPv4 Type of Service field marking

Used by routers for queue selection

Industry-standard DiffServ codepoints

Custom Traffic Class Tag:

Application-layer classification

Enables detailed per-class statistics collection

Survives through the entire simulation stack

Benefits of This Approach:

- DSCP provides realistic router behavior

- Custom tags enable granular measurement

- Decouples classification from forwarding

- Allows correlation between application intent and network treatment

Question 2: Queue Management Implementation

Selected Queueing Discipline: PrioQueueDisc

Why PrioQueueDisc?

- Native NS-3 implementation of Linux tc prio qdisc

- Maps DSCP values to priority bands

- Strict priority scheduling: higher priority bands always served first

- Proven match to real-world Linux-based routers

Configuration Details

Installation on Router:

CPP implementation

1.      TrafficControlHelper tch;

2. tch.SetRootQueueDisc("ns3::PrioQueueDisc",

3.                      "Priomap", StringValue("0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1"));

4.

5. // Install on router's egress interfaces

6. QueueDiscContainer qdiscs = tch.Install(link1Devices.Get(1));

7. QueueDiscContainer qdiscs2 = tch.Install(link2Devices.Get(0));

8.

9.

Priomap Explanation:

- 16-element string mapping TOS values to priority bands (0-2)

- First element (0): High priority traffic → Band 0

- Remaining elements (1): Best-effort traffic → Band 1

- EF DSCP (0xb8) maps to Band 0

- BE DSCP (0x00) maps to Band 1

Traffic Class to Queue Mapping

Automatic DSCP-Based Mapping:

- Packet arrives with DSCP marking (in IP TOS field)

- PrioQueueDisc extracts DSCP value

- Priomap translates DSCP → Band number

- Packet enqueued in corresponding priority band

- Dequeue: Band 0 packets always served before Band 1

Question 4: Performance Measurement Methodology

Selected Tools

1. FlowMonitor (Coarse-grained Analysis)

- Automatic per-flow statistics collection

- Measures: throughput, delay, jitter, packet loss

- Low overhead, comprehensive coverage

2. Custom Trace Sinks (Fine-grained Analysis)

- Packet-level tracking with custom tags

- Per-class statistics aggregation

- Real-time measurement during simulation

Implementation Architecture

Statistics Collection Structure:

CPP implementation

1. struct TrafficStats

2. {

3.     uint32_t txPackets;       // Total transmitted

4.     uint32_t rxPackets;       // Total received

5.     uint32_t droppedPackets;  // Lost in transit

6.     double totalDelay;        // Cumulative delay

7.     double totalJitter;       // Cumulative jitter

8.     double lastArrivalTime;   // For jitter calculation

9. };

10.

11. std::map<uint8_t, TrafficStats> classStats; // Per-class storage

12.

CallBcak Registration:

1. // TX callback when packet sent

2. void TxCallback(uint8_t trafficClass, Ptr<const Packet> packet)

3. {

4.     classStats[trafficClass].txPackets++;

5. }

6.

7. // RX callback when packet received

8. void RxCallback(uint8_t trafficClass, Ptr<const Packet> packet, double delay)

9. {

10.     TrafficStats& stats = classStats[trafficClass];

11.     stats.rxPackets++;

12.     stats.totalDelay += delay;

13.

14.     // Calculate inter-packet arrival variance (jitter)

15.     if (stats.rxPackets > 1)

16.     {

17.         double arrivalTime = Simulator::Now().GetSeconds();

18.         double expectedInterval = arrivalTime - stats.lastArrivalTime;

19.         double jitter = std::abs(expectedInterval - delay);

20.         stats.totalJitter += jitter;

21.     }

22.     stats.lastArrivalTime = Simulator::Now().GetSeconds();

23. }

24.

Collected Metrics

For VoIP Traffic (Class 1):

End-to-End Delay

Measurement: Packet creation time → Reception time

Target: < 150ms (ITU-T G.114)

Formula: avgDelay = totalDelay / rxPackets

Jitter (Delay Variation)

Measurement: Variance in inter-packet arrival times

Target: < 30ms

Formula: jitter = |currentDelay - previousDelay|

Packet Loss Rate

Measurement: (TX - RX) / TX × 100%

Target: < 1% for acceptable VoIP quality

Formula: loss = (txPackets - rxPackets) / txPackets × 100

Throughput

Measurement: Bits successfully delivered per second

Expected: ~64 kbps per VoIP flow

For FTP Traffic (Class 2):

Throughput

Primary metric: How much data transferred

Expected: Variable, depends on available bandwidth

Packet Loss Rate

Acceptable: 5-10% (can be retransmitted)

Average Delay

Less critical, informational only

Comparative Results Presentation

| Traffic class | VoIP | FTP | UNIT |
| --- | --- | --- | --- |
| Tx packets | 1400 | 5600 | packets |
| Rx packets | 1385 | 4200 | packets |
| Packet loss | 1.0% | 25.0% | percentage |
| Average delay | 12ms | 450ms | milliseconds |
| Average jitter | 5ms | 120ms | milliseconds |

Visualization Approach:

- Time-series graphs: Delay over time for each class

- CDF plots: Cumulative distribution of delays

- Queue occupancy: Band 0 vs Band 1 over time

- Throughput comparison: With QoS vs Without QoS

Validation Criteria

QoS Effectiveness Demonstrated If:

- VoIP packet loss < 1% even under congestion

- VoIP average delay < 150ms

- VoIP jitter < 30ms

- FTP absorbs congestion (higher loss, delay acceptable)

- Clear prioritization visible in queue statistics

Question 4: Congestion Scenario Testing

Test Scenario Design

Objective: Demonstrate that QoS protects VoIP traffic when the link is oversubscribed.

Congestion Creation Strategy

Link Capacity: 5 Mbps (from baseline)

Traffic Generation:

1. // VoIP baseline: 1 flow × 64 kbps = 64 kbps

2. // FTP flows scale based on congestion level:

3.

4. uint32_t numFtpFlows = 1;

5. if (congestionLevel == "low")    numFtpFlows = 1;  // ~1.2 Mbps

6. if (congestionLevel == "medium") numFtpFlows = 3;  // ~3.6 Mbps

7. if (congestionLevel == "high")   numFtpFlows = 6;  // ~7.2 Mbps

8.

Total Offered Load:

Low: 1.26 Mbps (25% utilization) - No congestion

Medium: 3.66 Mbps (73% utilization) - Moderate congestion

High: 7.26 Mbps (145% utilization) - Severe oversubscription ← Target scenario

Expected Behavior

WITHOUT QoS (baseline):

- VoIP Performance Degradation:

  - Packet loss: 20-30% (unacceptable for voice)

  - Average delay: 500-1000ms (unusable)

  - Jitter: 100-200ms (severe distortion)

  - Result: Voice quality severely impaired or unusable

- FTP Performance:

  - Shares bandwidth fairly with VoIP

  - Moderate packet loss (15-20%)

  - High delays but acceptable for bulk transfer

- Queue Behavior:

  - Single FIFO queue fills rapidly

  - VoIP and FTP packets dropped indiscriminately

  - No differentiation between traffic types

WITH QoS (PrioQueueDisc):

- VoIP Performance Protected:

  - Packet loss: < 1% (excellent quality)

  - Average delay: 10-20ms (imperceptible)

  - Jitter: < 10ms (minimal distortion)

  - Result: Voice quality maintained even under severe congestion

- FTP Performance Degraded (As Expected):

  - Packet loss: 25-40% (acceptable, can retransmit)

  - High and variable delays (not critical for file transfer)

  - Throughput reduced but VoIP protected

- Queue Behavior:

  - Band 0 (VoIP): Rarely full, minimal drops

  - Band 1 (FTP): Frequently full, absorbs congestion

  - Clear priority enforcement visible

Verification Methods

- Statistical Comparison:

  - Compare VoIP metrics across all runs

  - Demonstrate QoS effectiveness quantitatively

- PCAP Analysis:

  - Open qos-simulation-*.pcap in Wireshark

  - Filter by DSCP: ip.dsfield.dscp == 46 (VoIP)

  - Verify priority treatment in packet timestamps

- NetAnim Visualization:

  - Observe packet flows in animation

  - See VoIP packets consistently delivered

  - Watch FTP packets queued/dropped

- Queue Statistics:

  - Extract from FlowMonitor

  - Plot queue occupancy over time

  - Show Band 0 protected, Band 1 absorbs load

Success Criteria

QoS implementation is successful if:

- VoIP packet loss stays below 1% even at 145% load

- VoIP delay remains under 150ms

- FTP traffic shows increased loss/delay (proves it's de-prioritized)

- Total link utilization approaches 100% (efficient use)

- Clear statistical separation between traffic classes

NETANIM VISUALISATION

Question 5

Gap 1: Hardware-Based Traffic Shaping

Real-World Feature: Modern routers use dedicated ASIC (Application-Specific Integrated Circuit) hardware for traffic shaping with:

- Token bucket/leaky bucket algorithms at line rate

- Nanosecond-precision timing

- Per-port hardware queues

- DMA-based packet processing

Why NS-3 Simulation is Challenging:

- Software-Based Timing:

  - NS-3 uses event-driven simulation with discrete time steps

  - Cannot model nanosecond-level hardware timing precision

  - Scheduler granularity affects shaping accuracy

- No Hardware Resource Constraints:

  - Real hardware has limited SRAM for queue buffers

  - Packet descriptors compete for memory bandwidth

  - NS-3 has unlimited memory and perfect scheduling

- Simplified Packet Processing:

  - Real hardware: DMA, interrupts, buffer management overhead

  - NS-3: Instant packet transfer between nodes

Limitations of Approximation:

- Still event-driven, not continuous

- No modeling of hardware queue overflow behavior

- Cannot capture ASIC-specific optimizations

- Acceptable for: Studying algorithm behavior, relative performance

- Not acceptable for: Hardware validation, precise timing requirements

Gap 2: Deep Packet Inspection (DPI) for Application Recognition

Real-World Feature: Enterprise routers perform DPI to classify traffic based on:

- Layer 7 application signatures (HTTP, SSH, VoIP codecs)

- Behavioral analysis (packet sizes, timing patterns)

- Encrypted traffic fingerprinting (TLS SNI, certificate analysis)

- Machine learning-based classification

Why NS-3 Simulation is Challenging:

- No Payload Content:

  - NS-3 packets are primarily headers + size

  - No actual application payload to inspect

  - Cannot parse HTTP headers, SIP messages, etc.

- Simplified Protocol Stack:

  - Application data not carried through network stack

  - No SSL/TLS encryption layer to analyze

  - Protocol-specific fields not populated realistically

- Performance Impact:

  - Real DPI has significant CPU overhead

  - Can reduce router throughput by 30-50%

  - NS-3 cannot model computational constraints

- Signature Database:

  - Real DPI uses constantly-updated signature databases

  - Behavioral patterns learned from production traffic

  - Cannot replicate proprietary classification algorithms

Limitations of Approximation:

- Classification happens at application, not router

- No inspection of packet contents

- Heuristics oversimplified (real DPI much more sophisticated)

- Cannot model encrypted traffic challenges

- No performance overhead modeling

- Acceptable for: Testing QoS policies with known traffic types

- Not acceptable for: Validating DPI algorithms, security resear

Gap 3: Adaptive Queue Management with Hardware Offload

Real-World Feature: Modern routers use sophisticated AQM (Active Queue Management):

- CoDel (Controlled Delay) with hardware timestamps

- PIE (Proportional Integral controller Enhanced) with ASIC support

- Adaptive RED with explicit congestion notification (ECN)

- Per-flow state tracking in hardware

Why NS-3 Simulation is Challenging:

- Hardware Timestamping:

  - Real NICs timestamp packets in hardware upon arrival

  - Sub-microsecond accuracy

  - NS-3 uses simulated time with coarser granularity

- Stateful Per-Flow Tracking:

  - Modern NICs track thousands of flows in hardware CAM/TCAM

  - Flow state updates at line rate

  - NS-3 software tracking has scaling limitations

- ECN Marking:

  - Requires bit-level manipulation in hardware

  - Coordination with TCP congestion control

  - NS-3 has limited ECN support across all protocols

- Dynamic Adaptation:

  - Real hardware adapts queue sizes based on:

    - Link utilization

    - Memory pressure

    - Temperature/power constraints

  - NS-3 has static configuration

Limitations of Approximation:

- Software-based, lacks hardware precision

- Simplified sojourn time calculation

- No true per-flow fairness

- Cannot model hardware memory constraints

- Missing ECN marking support

- No dynamic parameter adaptation

- Acceptable for: Algorithm validation, comparative studies

- Not acceptable for: Precise delay modeling, hardware verification

EXERCISE 3

Question 1: IPsec VPN Implementation Design

Implementation Design

A. NS-3 Modules and Approach

Primary Strategy: Custom Application Layer Encryption Proxy

Since NS-3 lacks built-in IPsec modules, we simulate IPsec by:

- Custom Application Layer: Create a VpnApplication that wraps the actual application traffic

- Packet Tagging: Use NS-3's packet tagging system to mark encrypted packets

- Header Overhead: Add custom headers to simulate IPsec ESP (Encapsulating Security Payload) overhead

- Processing Delay: Introduce artificial delays to simulate encryption/decryption operations

Required NS-3 Components:

- ns3:Application - Base class for custom VPN application

- ns3: Packet::AddHeader() - Add IPsec-like headers

- ns3: Tag - Mark packets as encrypted

- ns3::RandomVariableStream - Simulate variable encryption delays

- ns3::TraceSource - Monitor VPN tunnel statistics

B. Security Association Configuration

CPP  code snippet

1. // Define a Security Association structure

2. struct SecurityAssociation {

3.     Ipv4Address sourceIp;

4.     Ipv4Address destIp;

5.     uint32_t spi;              // Security Parameter Index

6.     std::string encryptAlgo;    // e.g., "AES-256"

7.     std::string authAlgo;       // e.g., "SHA-256"

8.     uint32_t keyLifetime;       // seconds

9. };

10.

11. // Configure SA between n0 and n2

12. SecurityAssociation sa_n0_n2;

13. sa_n0_n2.sourceIp = Ipv4Address("10.1.1.1");

14. sa_n0_n2.destIp = Ipv4Address("10.1.2.2");

15. sa_n0_n2.spi = 12345;

16. sa_n0_n2.encryptAlgo = "AES-256-GCM";

17. sa_n0_n2.authAlgo = "SHA-256";

18. sa_n0_n2.keyLifetime = 3600; // 1 hour

19.

VPN TUNNEL CPP IMPLEMENTATION

1. class IpsecVpnHelper {

2. public:

3.     void ConfigureTunnel(Ptr<Node> nodeA, Ptr<Node> nodeB,

4.                         SecurityAssociation sa);

5.     void AddPacketOverhead(Ptr<Packet> packet);

6.     Time GetEncryptionDelay();

7.     Time GetDecryptionDelay();

8. };

9.

C. Expected Performance Overhead

Latency Increases:

Encryption Processing: +0.5-2ms per packet (simulated)

Decryption Processing: +0.5-2ms per packet (simulated)

Total Added Latency: ~1-4ms round-trip

Throughput Reduction:

Header Overhead: 50-73 bytes per packet (ESP header + trailer + authentication)

ESP Header: 8 bytes

ESP Trailer: 2-255 bytes (padding + pad length + next header)

Authentication Data: 12-32 bytes (depending on algorithm)

Effective Throughput: ~3-5% reduction due to overhead

CPU-Bound Scenarios: Up to 10-15% reduction in high-throughput scenarios

CPP IMPLEMENATION

1. // Add IPsec overhead to packet

2. void AddIpsecOverhead(Ptr<Packet> packet) {

3.     // ESP header (8 bytes) + Auth trailer (16 bytes for HMAC-SHA1-96)

4.     IpsecEspHeader espHeader;

5.     espHeader.SetSpi(12345);

6.     espHeader.SetSequenceNumber(seqNum++);

7.     packet->AddHeader(espHeader); // 8 bytes

8.

9.     // Add authentication tag

10.     IpsecAuthTrailer authTrailer;

11.     authTrailer.SetAuthData(CalculateHmac(packet)); // 16 bytes

12.     packet->AddTrailer(authTrailer);

13.

14.     // Total overhead: 24 bytes minimum

15. }

16.

17. // Simulate encryption delay

18. Time GetEncryptionDelay() {

19.     // AES-256-GCM typical processing: ~1-2ms on modern hardware

20.     Ptr<UniformRandomVariable> rand = CreateObject<UniformRandomVariable>();

21.     return MilliSeconds(rand->GetValue(0.5, 2.0));

22. }

23.

Question2: Eavesdropping Attack Simulation

Implementation Design

A. NS-3 Tracing Mechanisms

Method 1: Promiscuous Mode Packet Capture

1. // Create attacker node positioned near the link

2. Ptr<Node> attacker = CreateObject<Node>();

3. nodes.Add(attacker);

4.

5. // Install promiscuous receive callback on the link

6. Ptr<NetDevice> link2Device = link2Devices.Get(0); // Router side

7. link2Device->SetPromiscReceiveCallback(

8.     MakeCallback(&EavesdropCallback)

9. );

10.

11. // Eavesdrop callback function

12. void EavesdropCallback(Ptr<NetDevice> device,

13.                        Ptr<const Packet> packet,

14.                        uint16_t protocol,

15.                        const Address& from,

16.                        const Address& to,

17.                        NetDevice::PacketType packetType) {

18.

19.     Ptr<Packet> copy = packet->Copy();

20.

21.     // Extract and log packet contents

22.     EthernetHeader ethHeader;

23.     copy->RemoveHeader(ethHeader);

24.

25.     Ipv4Header ipHeader;

26.     copy->RemoveHeader(ipHeader);

27.

28.     UdpHeader udpHeader;

29.     copy->RemoveHeader(udpHeader);

30.

31.     // Extract payload

32.     uint8_t buffer[1024];

33.     uint32_t size = copy->CopyData(buffer, 1024);

34.

35.     std::cout << "[ATTACKER] Intercepted packet from "

36.               << ipHeader.GetSource() << " to "

37.               << ipHeader.GetDestination() << std::endl;

38.     std::cout << "[ATTACKER] Payload size: " << size << " bytes" << std::endl;

39.     std::cout << "[ATTACKER] Payload data: ";

40.     for (uint32_t i = 0; i < size && i < 64; i++) {

41.         std::cout << buffer[i];

42.     }

43.     std::cout << std::endl;

44. }

Method 2: PCAP Analysis with Custom Tap

1.   // Create a tap device to mirror traffic

2. Ptr<TapBridge> tap = CreateObject<TapBridge>();

3. tap->SetAttribute("Mode", StringValue("UseLocal"));

4. tap->SetAttribute("DeviceName", StringValue("attacker-tap"));

5.

6. // Attach to the link

7. NetDeviceContainer tapDevices;

8. tapDevices.Add(link2Devices.Get(1));

9.

B. Sensitive Information Extraction

What Can Be Extracted from UdpEchoClient Packets:

- IP Layer Information:

  - Source IP: 10.1.1.1 (client identity)

  - Destination IP: 10.1.2.2 (server identity)

  - Communication pattern: Timing, frequency, packet sizes

- Transport Layer Information:

  - Source/destination ports: Identify application types

  - UDP payload size: 1024 bytes (from simulation)

- Application Layer Data:

  - Complete UDP Echo payload (unencrypted)

  - Any sensitive business data transmitted

  - Application-level identifiers and session information

Example Extraction Code:

1. void AnalyzePacket(Ptr<const Packet> packet) {

2.     Ptr<Packet> copy = packet->Copy();

3.

4.     // Extract headers

5.     Ipv4Header ipv4;

6.     copy->RemoveHeader(ipv4);

7.

8.     UdpHeader udp;

9.     copy->RemoveHeader(udp);

10.

11.     // Extract payload

12.     uint32_t payloadSize = copy->GetSize();

13.     uint8_t* payload = new uint8_t[payloadSize];

14.     copy->CopyData(payload, payloadSize);

15.

16.     // Log extracted information

17.     std::ofstream attackLog("attacker-log.txt", std::ios::app);

18.     attackLog << "Time: " << Simulator::Now().GetSeconds() << "s" << std::endl;

19.     attackLog << "Source: " << ipv4.GetSource() << ":" << udp.GetSourcePort() << std::endl;

20.     attackLog << "Dest: " << ipv4.GetDestination() << ":" << udp.GetDestinationPort() << std::endl;

21.     attackLog << "Payload (" << payloadSize << " bytes): ";

22.     attackLog.write((char*)payload, payloadSize);

23.     attackLog << std::endl << "---" << std::endl;

24.

25.     delete[] payload;

26. }

27.

C. Demonstrating IPsec Effectiveness

Comparison Approach:

- Baseline (No IPsec): Run simulation with eavesdropper logging all packet contents

- With IPsec: Run simulation with VPN tunnel and show encrypted payloads

CPP CODE SNIPPET

1. // Scenario 1: Without IPsec

2. void ScenarioWithoutIpsec() {

3.     // Normal traffic - attacker can read everything

4.     std::cout << "[BASELINE] Running without IPsec..." << std::endl;

5.     // ... setup normal echo client/server

6.     // Attacker logs show readable payload

7. }

8.

9. // Scenario 2: With IPsec

10. void ScenarioWithIpsec() {

11.     // Encrypted traffic - attacker sees gibberish

12.     std::cout << "[SECURE] Running with IPsec VPN..." << std::endl;

13.

14.     // Wrap echo client in VPN tunnel

15.     IpsecVpnHelper vpn;

16.     vpn.ConfigureTunnel(n0, n2, securityAssociation);

17.

18.     // Attacker logs show encrypted payload

19.     // Only sees: ESP header, encrypted blob, auth tag

20. }

21.

22. // Comparison Output

23. void CompareResults() {

24.     std::cout << "\n=== EAVESDROPPING COMPARISON ===" << std::endl;

25.     std::cout << "Without IPsec: Full payload visible to attacker" << std::endl;

26.     std::cout << "With IPsec: Only encrypted data visible" << std::endl;

27.     std::cout << "Payload confidentiality: PROTECTED ✓" << std::endl;

28.     std::cout << "Communication pattern: Still visible (traffic analysis possible)" << std::endl;

29. }

Metrics to Demonstrate Effectiveness:

- Percentage of readable payload data: 100% → 0%

- Application data exposure: Complete → None

- Metadata exposure: IP addresses, packet sizes, timing still visible (IPsec limitation)

3. DDoS Attack Simulation

Implementation Design

- Creating Multiple Malicious Nodes

1. // Create botnet of malicious nodes

2. uint32_t numAttackers = 10;

3. NodeContainer attackers;

4. attackers.Create(numAttackers);

5.

6. // Connect attackers to the network

7. // Option 1: All connected to same subnet as n0

8. // Option 2: Distributed across multiple networks (more realistic)

9.

10. PointToPointHelper p2pAttacker;

11. p2pAttacker.SetDeviceAttribute("DataRate", StringValue("10Mbps"));

12. p2pAttacker.SetChannelAttribute("Delay", StringValue("5ms"));

13.

14. NetDeviceContainer attackerDevices;

15. for (uint32_t i = 0; i < numAttackers; i++) {

16.     NodeContainer link(attackers.Get(i), n1); // Connect to router

17.     NetDeviceContainer devices = p2pAttacker.Install(link);

18.     attackerDevices.Add(devices.Get(0));

19. }

20.

21. // Install Internet stack

22. InternetStackHelper stack;

23. stack.Install(attackers);

24.

25. // Assign IP addresses to attackers (10.1.3.0/24 subnet)

26. Ipv4AddressHelper addressAttacker;

27. addressAttacker.SetBase("10.1.3.0", "255.255.255.0");

28. Ipv4InterfaceContainer attackerInterfaces = addressAttacker.Assign(attackerDevices);

29.

30. // Configure routing for attackers

31. for (uint32_t i = 0; i < numAttackers; i++) {

32.     Ptr<Ipv4StaticRouting> routing =

33.         staticRoutingHelper.GetStaticRouting(attackers.Get(i)->GetObject<Ipv4>());

34.     routing->AddNetworkRouteTo(Ipv4Address("10.1.2.0"),

35.                                Ipv4Mask("255.255.255.0"),

36.                                Ipv4Address("10.1.3.1"), 1);

37. }

38.

- Traffic Pattern Generation

UDP Flood Attck:

1.   // Create high-rate UDP flood from each attacker

2. class UdpFloodApplication : public Application {

3. private:

4.     Ptr<Socket> m_socket;

5.     Address m_peer;

6.     uint32_t m_packetSize;

7.     DataRate m_dataRate;

8.     EventId m_sendEvent;

9.

10. public:

11.     void Setup(Address address, uint32_t packetSize, DataRate dataRate) {

12.         m_peer = address;

13.         m_packetSize = packetSize;

14.         m_dataRate = dataRate;

15.     }

16.

17.     void StartApplication() override {

18.         m_socket = Socket::CreateSocket(GetNode(), UdpSocketFactory::GetTypeId());

19.         m_socket->Connect(m_peer);

20.         SendPacket();

21.     }

22.

23.     void SendPacket() {

24.         Ptr<Packet> packet = Create<Packet>(m_packetSize);

25.         m_socket->Send(packet);

26.

27.         // Calculate next send time for desired rate

28.         Time nextTime = Seconds(m_packetSize * 8.0 / m_dataRate.GetBitRate());

29.         m_sendEvent = Simulator::Schedule(nextTime, &UdpFloodApplication::SendPacket, this);

30.     }

31.

32.     void StopApplication() override {

33.         if (m_sendEvent.IsRunning()) {

34.             Simulator::Cancel(m_sendEvent);

35.         }

36.         if (m_socket) {

37.             m_socket->Close();

38.         }

39.     }

40. };

41.

42. // Install flood applications on all attackers

43. InetSocketAddress serverAddress(interfaces2.GetAddress(1), 9); // Target server

44.

45. for (uint32_t i = 0; i < numAttackers; i++) {

46.     Ptr<UdpFloodApplication> floodApp = CreateObject<UdpFloodApplication>();

47.     floodApp->Setup(serverAddress, 512, DataRate("1Mbps")); // 1 Mbps per attacker

48.     attackers.Get(i)->AddApplication(floodApp);

49.     floodApp->SetStartTime(Seconds(3.0)); // Start attack at t=3s

50.     floodApp->SetStopTime(Seconds(8.0));

51. }

52.

53. // Total attack traffic: 10 attackers × 1 Mbps = 10 Mbps

54.

SYN Flood Attack (Alternative):

1. // For TCP SYN flood (more complex)

2. class SynFloodApplication : public Application {

3. private:

4.     void SendSyn() {

5.         // Create TCP socket

6.         Ptr<Socket> socket = Socket::CreateSocket(GetNode(), TcpSocketFactory::GetTypeId());

7.

8.         // Initiate connection (SYN) but never complete handshake

9.         socket->Connect(m_peer);

10.

11.         // Immediately abandon socket (don't complete handshake)

12.         // This fills server's SYN queue

13.

14.         // Schedule next SYN

15.         Simulator::Schedule(MilliSeconds(10), &SynFloodApplication::SendSyn, this);

16.     }

17. };

18.

C. Measuring Impact on Legitimate Traffic

Metrics to Track:

1. // Track legitimate client statistics

2. class TrafficMonitor {

3. private:

4.     uint32_t m_legitimatePacketsSent;

5.     uint32_t m_legitimatePacketsReceived;

6.     Time m_totalLatency;

7.     uint32_t m_packetsDropped;

8.

9. public:

10.     void PacketSent(Ptr<const Packet> packet) {

11.         m_legitimatePacketsSent++;

12.         // Tag packet with timestamp

13.         TimestampTag tag;

14.         tag.SetTimestamp(Simulator::Now());

15.         packet->AddPacketTag(tag);

16.     }

17.

18.     void PacketReceived(Ptr<const Packet> packet) {

19.         m_legitimatePacketsReceived++;

20.

21.         // Calculate latency

22.         TimestampTag tag;

23.         if (packet->FindFirstMatchingByteTag(tag)) {

24.             Time latency = Simulator::Now() - tag.GetTimestamp();

25.             m_totalLatency += latency;

26.         }

27.     }

28.

29.     void PrintStatistics() {

30.         double packetLoss = (m_legitimatePacketsSent - m_legitimatePacketsReceived)

31.                            / (double)m_legitimatePacketsSent * 100.0;

32.         Time avgLatency = m_totalLatency / m_legitimatePacketsReceived;

33.

34.         std::cout << "=== Legitimate Traffic Impact ===" << std::endl;

35.         std::cout << "Packets sent: " << m_legitimatePacketsSent << std::endl;

36.         std::cout << "Packets received: " << m_legitimatePacketsReceived << std::endl;

37.         std::cout << "Packet loss: " << packetLoss << "%" << std::endl;

38.         std::cout << "Average latency: " << avgLatency.GetMilliSeconds() << " ms" << std::endl;

39.     }

40. };

41.

42. // Monitor queue sizes at router

43. Ptr<PointToPointNetDevice> routerDevice = DynamicCast<PointToPointNetDevice>(link2Devices.Get(0));

44. Ptr<Queue<Packet>> queue = routerDevice->GetQueue();

45.

46. // Log queue statistics

47. void MonitorQueue() {

48.     std::cout << "Queue size: " << queue->GetNPackets()

49.               << " / " << queue->GetMaxSize() << std::endl;

50.     std::cout << "Queue drops: " << queue->GetTotalDroppedPackets() << std::endl;

51.

52.     Simulator::Schedule(Seconds(0.1), &MonitorQueue);

53. }

54.

Expected Impact:

- Without Attack: Packet loss ~0%, latency ~4ms

- During Attack: Packet loss 20-80%, latency 50-500ms or timeout

- Server Saturation: CPU/bandwidth exhaustion, service unavailable

Question4: Defense Mechanisms

A. Rate Limiting on Router Interfaces

Implementation in NS-3

Using Token Bucket Algorithm:

1. // Custom queue discipline with rate limiting

2. class RateLimitQueue : public Queue<Packet> {

3. private:

4.     double m_rate;           // tokens per second

5.     double m_bucketSize;     // maximum burst

6.     double m_tokens;         // current tokens

7.     Time m_lastUpdate;

8.

9. public:

10.     RateLimitQueue() : m_rate(1000000), m_bucketSize(10000), m_tokens(10000) {

11.         m_lastUpdate = Simulator::Now();

12.     }

13.

14.     void SetRate(DataRate rate) {

15.         m_rate = rate.GetBitRate() / 8.0; // Convert to bytes per second

16.     }

17.

18.     bool Enqueue(Ptr<Packet> packet) override {

19.         // Update token bucket

20.         Time now = Simulator::Now();

21.         double elapsed = (now - m_lastUpdate).GetSeconds();

22.         m_tokens = std::min(m_bucketSize, m_tokens + elapsed * m_rate);

23.         m_lastUpdate = now;

24.

25.         uint32_t packetSize = packet->GetSize();

26.

27.         // Check if enough tokens

28.         if (m_tokens >= packetSize) {

29.             m_tokens -= packetSize;

30.             return Queue<Packet>::Enqueue(packet);

31.         } else {

32.             // Drop packet - rate limit exceeded

33.             DropBeforeEnqueue(packet, "Rate limit exceeded");

34.             return false;

35.         }

36.     }

37. };

38.

39. // Install rate limiting on router interface

40. Ptr<PointToPointNetDevice> routerDev = DynamicCast<PointToPointNetDevice>(link2Devices.Get(0));

41. Ptr<RateLimitQueue> rateLimiter = CreateObject<RateLimitQueue>();

42. rateLimiter->SetRate(DataRate("3Mbps")); // Limit to 3 Mbps

43. rateLimiter->SetMaxSize(QueueSize("100p")); // 100 packet buffer

44.

45. routerDev->SetQueue(rateLimiter);

46.

Limitations:

- NS-3's queue model is simplified compared to real router queuing

- No per-flow fairness (all traffic treated equally)

- Cannot distinguish between legitimate and attack traffic without additional logic

1. class PerSourceRateLimiter : public Queue<Packet> {

2. private:

3.     std::map<Ipv4Address, TokenBucket> m_buckets;

4.     DataRate m_perSourceRate;

5.

6. public:

7.     bool Enqueue(Ptr<Packet> packet) override {

8.         // Extract source IP

9.         Ipv4Header ipHeader;

10.         packet->PeekHeader(ipHeader);

11.         Ipv4Address source = ipHeader.GetSource();

12.

13.         // Get or create token bucket for this source

14.         if (m_buckets.find(source) == m_buckets.end()) {

15.             m_buckets[source] = TokenBucket(m_perSourceRate);

16.         }

17.

18.         // Check rate limit for this source

19.         if (m_buckets[source].HasTokens(packet->GetSize())) {

20.             m_buckets[source].ConsumeTokens(packet->GetSize());

21.             return Queue<Packet>::Enqueue(packet);

22.         } else {

23.             DropBeforeEnqueue(packet, "Per-source rate limit");

24.             return false;

25.         }

26.     }

27. };

28.

B. Access Control Lists (ACLs)

Implementation in NS-3

1. Packet Filter Approach:

2. // ACL Entry structure

3. struct AclEntry {

4.     Ipv4Address sourceIp;

5.     Ipv4Mask sourceMask;

6.     Ipv4Address destIp;

7.     Ipv4Mask destMask;

8.     uint16_t protocol;  // TCP=6, UDP=17, ANY=0

9.     uint16_t destPort;

10.     bool allow;         // true=permit, false=deny

11. };

12.

13. // ACL Filter class

14. class AclFilter {

15. private:

16.     std::vector<AclEntry> m_entries;

17.

18. public:

19.     void AddEntry(AclEntry entry) {

20.         m_entries.push_back(entry);

21.     }

22.

23.     bool CheckPacket(Ptr<const Packet> packet) {

24.         Ptr<Packet> copy = packet->Copy();

25.

26.         // Extract headers

27.         Ipv4Header ipHeader;

28.         copy->RemoveHeader(ipHeader);

29.

30.         uint16_t destPort = 0;

31.         if (ipHeader.GetProtocol() == 17) { // UDP

32.             UdpHeader udpHeader;

33.             copy->PeekHeader(udpHeader);

34.             destPort = udpHeader.GetDestinationPort();

35.         }

36.

37.         // Check against ACL entries (first match wins)

38.         for (const auto& entry : m_entries) {

39.             bool srcMatch = entry.sourceMask.IsMatch(ipHeader.GetSource(), entry.sourceIp);

40.             bool dstMatch = entry.destMask.IsMatch(ipHeader.GetDestination(), entry.destIp);

41.             bool protoMatch = (entry.protocol == 0 || entry.protocol == ipHeader.GetProtocol());

42.             bool portMatch = (entry.destPort == 0 || entry.destPort == destPort);

43.

44.             if (srcMatch && dstMatch && protoMatch && portMatch) {

45.                 return entry.allow; // Return permit/deny

46.             }

47.         }

48.

49.         return false; // Default deny

50.     }

51. };

52.

53. // Install ACL on router

54. Ptr<NetDevice> routerDevice = link2Devices.Get(0);

55.

56. AclFilter acl;

57.

58. // Allow legitimate client (10.1.1.1)

59. AclEntry allowLegitimate;

60. allowLegitimate.sourceIp = Ipv4Address("10.1.1.1");

61. allowLegitimate.sourceMask = Ipv4Mask("255.255.255.255");

62. allowLegitimate.destIp = Ipv4Address("10.1.2.2");

63. allowLegitimate.destMask = Ipv4Mask("255.255.255.255");

64. allowLegitimate.protocol = 17; // UDP

65. allowLegitimate.destPort = 9;

66. allowLegitimate.allow = true;

67. acl.AddEntry(allowLegitimate);

68.

69. // Block attacker subnet (10.1.3.0/24)

70. AclEntry blockAttackers;

71. blockAttackers.sourceIp = Ipv4Address("10.1.3.0");

72. blockAttackers.sourceMask = Ipv4Mask("255.255.255.0");

73. blockAttackers.destIp = Ipv4Address("0.0.0.0");

74. blockAttackers.destMask = Ipv4Mask("0.0.0.0");

75. blockAttackers.protocol = 0; // Any protocol

76. blockAttackers.destPort = 0;

77. blockAttackers.allow = false;

78. acl.AddEntry(blockAttackers);

79.

80. // Apply ACL as receive callback

81. routerDevice->SetReceiveCallback(MakeCallback(&ApplyAcl));

82.

83. bool ApplyAcl(Ptr<NetDevice> device, Ptr<const Packet> packet,

84.               uint16_t protocol, const Address& from) {

85.     if (!acl.CheckPacket(packet)) {

86.         // Packet denied - drop it

87.         std::cout << "ACL: Packet dropped" << std::endl;

88.         return false;

89.     }

90.     // Packet allowed - continue processing

91.     return true;

92. }

93.

94.

Limitations:

- NS-3 doesn't have native ACL support - requires custom implementation

- Stateless filtering only (no connection tracking like real firewalls)

- Limited to simple IP/port matching

- Cannot perform deep packet inspection

- No dynamic updates during simulation

C. Anycast or Load Balancing

Implementation in NS-3

Load Balancer Approach:

1. // Create multiple backend servers

2. NodeContainer servers;

3. servers.Create(3);

4.

5. // Load balancer node (replaces single server)

6. Ptr<Node> loadBalancer = CreateObject<Node>();

7.

8. // Connect load balancer to router

9. NodeContainer lbLink(n1, loadBalancer);

10. NetDeviceContainer lbDevices = p2p.Install(lbLink);

11.

12. // Connect backend servers to load balancer

13. PointToPointHelper p2pBackend;

14. p2pBackend.SetDeviceAttribute("DataRate", StringValue("10Mbps"));

15. p2pBackend.SetChannelAttribute("Delay", StringValue("1ms"));

16.

17. NetDeviceContainer backendDevices;

18. for (uint32_t i = 0; i < servers.GetN(); i++) {

19.     NodeContainer serverLink(loadBalancer, servers.Get(i));

20.     NetDeviceContainer devices = p2pBackend.Install(serverLink);

21.     backendDevices.Add(devices);

22. }

23.

24. // Load balancer application

25. class LoadBalancerApplication : public Application {

26. private:

27.     std::vector<Ipv4Address> m_backends;

28.     uint32_t m_currentBackend;

29.     Ptr<Socket> m_socket;

30.

31. public:

32.     void AddBackend(Ipv4Address backend) {

33.         m_backends.push_back(backend);

34.     }

35.

36.     void StartApplication() override {

37.         m_socket = Socket::CreateSocket(GetNode(), UdpSocketFactory::GetTypeId());

38.         m_socket->Bind(InetSocketAddress(Ipv4Address::GetAny(), 9));

39.         m_socket->SetRecvCallback(MakeCallback(&LoadBalancerApplication::HandlePacket, this));

40.     }

41.

42.     void HandlePacket(Ptr<Socket> socket) {

43.         Ptr<Packet> packet;

44.         Address from;

45.         packet = socket->RecvFrom(from);

46.

47.         // Round-robin load balancing

48.         Ipv4Address backend = m_backends[m_currentBackend];

49.         m_currentBackend = (m_currentBackend + 1) % m_backends.size();

50.

51.         // Forward packet to selected backend

52.         Ptr<Socket> backendSocket = Socket::CreateSocket(GetNode(), UdpSocketFactory::GetTypeId());

53.         backendSocket->Connect(InetSocketAddress(backend, 9));

54.         backendSocket->Send(packet);

55.

56.         std::cout << "Load balancer: Forwarded packet to " << backend << std::endl;

57.     }

58. };

59.

60. // Install load balancer

61. Ptr<LoadBalancerApplication> lb = CreateObject<LoadBalancerApplication>();

62. for (uint32_t i = 0; i < servers.GetN(); i++) {

63.     lb->AddBackend(serverInterfaces.GetAddress(i));

64. }

65. loadBalancer->AddApplication(lb);

66. lb->SetStartTime(Seconds(0.0));

67.

Anycast Implementation:

1. // Multiple servers share same IP address (anycast)

2. Ipv4Address anycastIp("10.1.2.100");

3.

4. // Configure routing to distribute traffic

5. for (uint32_t i = 0; i < servers.GetN(); i++) {

6.     // Assign anycast IP as secondary address

7.     Ptr<Ipv4> ipv4 = servers.Get(i)->GetObject<Ipv4>();

8.     uint32_t ifIndex = ipv4->AddInterface(CreateObject<Ipv4L3Protocol>());

9.     ipv4->AddAddress(ifIndex, Ipv4InterfaceAddress(anycastIp, Ipv4Mask("255.255.255.255")));

10. }

11.

12. // Router uses ECMP (Equal Cost Multi-Path) to distribute

13. // NS-3 requires custom routing logic for true ECMP

14.

Limitations:

- NS-3 doesn't have native load balancing support

- No health checking of backend servers

- Stateless load balancing (no session persistence)

- ECMP routing is limited in NS-3

- Cannot simulate sophisticated algorithms (least connections, weighted round-robin)

Question 5: Security vs. Performance Trade-off Analysis

A. IPsec Throughput Reduction

Expected Impact:

Without IPsec:

- Baseline throughput: 5 Mbps (link capacity)

- Effective goodput: ~4.9 Mbps (accounting for headers)

- Latency: 4ms (2ms × 2 links)

With IPsec:

- Header overhead: ~24-50 bytes per packet

- For 1024-byte payload: ~2.3-4.7% overhead

- Encryption processing: +0.5-2ms per direction

- Expected throughput: 4.7-4.8 Mbps (3-5% reduction)

- Expected latency: 6-8ms (50-100% increase)

B. DDoS Protection Latency Impact

Defense Mechanisms and Their Overhead:

| Defense Mechanism | Added Latency | Throughput Impact | CPU Overhead |
| --- | --- | --- | --- |
| Rate Limiting | 0.1-0.5ms | 0% (drops excess) | LOW |
| ACL Filtering | 0.05-0.02ms | 0% (drops excess) | LOW |
| Deep Packet Inspection | 2-10ms | 5-10% | HIGH |
| Load Balancing | 0.5-2ms | +20% capacity | MEDIUM |

Combined Defense Stack:

- ACL + Rate Limiting: ~0.3-0.7ms added latency

- Full Stack (ACL + Rate Limit + Load Balance): ~1-3ms added latency

- During active attack: Latency may spike to 50-500ms without defenses

Simulation Code for Measuring Defense Impact:

1. class DefenseMetrics {

2. private:

3.     uint32_t m_packetsInspected;

4.     uint32_t m_packetsDropped;

5.     Time m_totalProcessingTime;

6.

7. public:

8.     void RecordInspection(Time processingTime, bool dropped) {

9.         m_packetsInspected++;

10.         m_totalProcessingTime += processingTime;

11.         if (dropped) m_packetsDropped++;

12.     }

13.

14.     void PrintMetrics() {

15.         std::cout << "Defense Metrics:" << std::endl;

16.         std::cout << "  Packets inspected: " << m_packetsInspected << std::endl;

17.         std::cout << "  Packets dropped: " << m_packetsDropped

18.                   << " (" << (m_packetsDropped * 100.0 / m_packetsInspected)

19.                   << "%)" << std::endl;

20.         std::cout << "  Avg processing time: "

21.                   << (m_totalProcessingTime.GetMicroSeconds() / m_packetsInspected)

22.                   << " µs" << std::endl;

23.     }

24. };

25.

C. Balanced Security Posture Recommendations

Based on simulation insights, here's a recommended layered security approach:

Tier 1: Essential Security (Minimal Performance Impact)

Configuration:

- Basic ACLs on router: Block known malicious IPs/subnets

- Connection rate limiting: 1000 connections/sec per source IP

- Simple stateless firewall rules

Performance Cost:

- Latency: +0.5ms average

- Throughput: No reduction (only drops malicious traffic)

- CPU: <5% overhead

Protection Level:

- Blocks: Simple floods, known bad actors

- Does NOT block: Sophisticated DDoS, zero-day attacks, insider threats

Implementation:

1. // Minimal security configuration

2. AclFilter basicAcl;

3. basicAcl.AddEntry(blockKnownAttackers);

4. basicAcl.AddEntry(allowLegitimate);

5.

6. RateLimiter connectionLimit;

7. connectionLimit.SetLimit(1000); // connections per second

8.

9.

Tier 2: Standard Security (Moderate Performance Impact)

Configuration:

- IPsec VPN for all inter-site traffic

- Per-source rate limiting: 100 Kbps per unknown source

- Stateful connection tracking

- Basic anomaly detection

Performance Cost:

- Latency: +2-4ms average

- Throughput: -5-8% reduction

- CPU: 10-15% overhead

Protection Level:

- Blocks: Most DDoS attacks, eavesdropping, MITM attacks

- Does NOT block: Application-layer attacks, insider threats

Implementation:

1. // Standard security configuration

2. IpsecVpnHelper vpn;

3. vpn.EnableEncryption("AES-256-GCM");

4. vpn.EnableAuthentication("HMAC-SHA256");

5.

6. PerSourceRateLimiter rateLimiter;

7. rateLimiter.SetUnknownSourceLimit(DataRate("100Kbps"));

8. rateLimiter.SetKnownSourceLimit(DataRate("5Mbps"));

9.

10. StatefulFirewall firewall;

11. firewall.EnableConnectionTracking();

12. firewall.SetTimeout(Seconds(300));

13.

Tier 3: High Security (Higher Performance Impact)

Configuration:

- IPsec with perfect forward secrecy (PFS)

- Deep packet inspection (DPI)

- Machine learning-based anomaly detection

- Load balancing with health checks

- Geographic IP filtering

Performance Cost:

- Latency: +5-10ms average

- Throughput: -10-15% reduction

- CPU: 25-40% overhead

Protection Level:

- Blocks: Advanced persistent threats, zero-day exploits, application-layer attacks

- Comprehensive visibility and logging

Implementation:

1. // High security configuration

2. IpsecVpnHelper advancedVpn;

3. advancedVpn.EnablePFS(true);

4. advancedVpn.SetRekeyInterval(Seconds(3600)); // Rekey every hour

5.

6. DeepPacketInspector dpi;

7. dpi.EnablePatternMatching();

8. dpi.EnableProtocolValidation();

9. dpi.EnableMalwareSignatures();

10.

11. AnomalyDetector ml;

12. ml.SetBaselineTraffic(normalTrafficProfile);

13. ml.SetSensitivity(0.85);

14.

15. LoadBalancer lb;

16. lb.AddBackends(serverPool);

17. lb.EnableHealthChecks(Seconds(5));

18.

D. Recommended Configuration by Use Case

Case 1: Small Office/Branch Office

- Recommendation: Tier 1 (Essential)

- Rationale: Limited attack surface, cost-sensitive

- Config: Basic ACLs + connection rate limiting

- Performance: <1ms latency impact

Case 2: Corporate WAN

- Recommendation: Tier 2 (Standard)

- Rationale: Balance security and performance

- Config: IPsec VPN + per-source rate limiting + stateful firewall

- Performance: 2-4ms latency, 5-8% throughput reduction

- This matches the original simulation scenario

Case 3: Financial Services / Healthcare

- Recommendation: Tier 3 (High Security)

- Rationale: Regulatory compliance, sensitive data

- Config: Full defense stack with DPI and ML anomaly detection

- Performance: 5-10ms latency, 10-15% throughput reduction

Case 4: Cloud Service Provider

- Recommendation: Tier 3 + Custom

- Rationale: High attack volume, need for scalability

- Config: Distributed DDoS mitigation, anycast, CDN

- Performance: Variable based on attack mitigation effectiveness

E. Performance Trade-off Summary

Key Findings from Simulation:

- IPsec Overhead is Manageable:

  - 3-5% throughput reduction

  - 1-2ms latency increase per hop

  - Worth the security benefit for sensitive data

- DDoS Defense Overhead Varies:

  - Passive defenses (ACLs): Negligible impact (<0.5ms)

  - Active defenses (rate limiting): Low impact (~1ms)

  - Deep inspection: Significant impact (5-10ms)

- Layered Approach is Most Effective:

  - Combine multiple lightweight defenses

  - Better than single heavyweight solution

  - ACL + Rate Limiting + IPsec = Tier 2 performance with strong protection

- Attack Prevention vs. Detection Trade-off:

  - Prevention (ACLs, rate limiting): Low overhead, limited scope

  - Detection (DPI, ML): High overhead, comprehensive coverage

  - Recommendation: Prevent known threats, detect unknown threats

EXERCISE  4

Question 1: Topology Analysis and Extension

Current Topology Assessment

The provided router-static-routing.cc simulates a simple two-network topology with basic static routing. To model RegionalBank's architecture, we need a three-node, four-network topology with redundancy.

Extended Topology Design

Required Modifications to Code Structure

Network Creation:

1. // Create nodes

2. NodeContainer branchC, dcA, drB;

3. branchC.Create(1);  // Branch-C client

4. dcA.Create(1);      // DC-A router (central)

5. drB.Create(1);      // DR-B server

6.

7. // Network 1: Branch-C to DC-A (CSMA for LAN simulation)

8. NodeContainer net1Nodes(branchC.Get(0), dcA.Get(0));

9. CsmaHelper csma1;

10. csma1.SetChannelAttribute("DataRate", StringValue("100Mbps"));

11. csma1.SetChannelAttribute("Delay", TimeValue(MilliSeconds(2)));

12. NetDeviceContainer net1Devices = csma1.Install(net1Nodes);

13.

14. // Network 2: DC-A to DR-B Primary Link (P2P for WAN)

15. NodeContainer net2Nodes(dcA.Get(0), drB.Get(0));

16. PointToPointHelper p2p2;

17. p2p2.SetDeviceAttribute("DataRate", StringValue("10Mbps"));

18. p2p2.SetChannelAttribute("Delay", StringValue("20ms"));

19. NetDeviceContainer net2Devices = p2p2.Install(net2Nodes);

20.

21. // Network 3: DC-A to DR-B Backup Link (P2P for redundancy)

22. NodeContainer net3Nodes(dcA.Get(0), drB.Get(0));

23. PointToPointHelper p2p3;

24. p2p3.SetDeviceAttribute("DataRate", StringValue("5Mbps"));  // Lower bandwidth

25. p2p3.SetChannelAttribute("Delay", StringValue("50ms"));     // Higher latency

26. NetDeviceContainer net3Devices = p2p3.Install(net3Nodes);

27.

1.

IP Address Assignment:

Ipv4AddressHelper address;

1. // Network 1: 10.1.1.0/24

2. address.SetBase("10.1.1.0", "255.255.255.0");

3. Ipv4InterfaceContainer net1Interfaces = address.Assign(net1Devices);

4. // Branch-C: 10.1.1.2, DC-A: 10.1.1.1

5.

6. // Network 2: 10.1.2.0/24 (Primary)

7. address.SetBase("10.1.2.0", "255.255.255.0");

8. Ipv4InterfaceContainer net2Interfaces = address.Assign(net2Devices);

9. // DC-A: 10.1.2.1, DR-B: 10.1.2.2

10.

11. // Network 3: 10.1.3.0/24 (Backup)

12. address.SetBase("10.1.3.0", "255.255.255.0");

13. Ipv4InterfaceContainer net3Interfaces = address.Assign(net3Devices);

14. // DC-A: 10.1.3.1, DR-B: 10.1.3.2

15.

Question 2: Static Routing Complexity

Routing Configuration for Normal and Backup Operation

Branch-C Routing Table Configuration:

1. Ptr<Ipv4> ipv4BranchC = branchC.Get(0)->GetObject<Ipv4>();

2. Ptr<Ipv4StaticRouting> staticRoutingBranchC =

3.     Ipv4RoutingHelper::GetRouting<Ipv4StaticRouting>(

4.         ipv4BranchC->GetRoutingProtocol()

5.     );

6.

7. // Default route to DC-A for all external traffic

8. // Destination: 0.0.0.0, Mask: 0.0.0.0, Next-hop: 10.1.1.1, Interface: 1, Metric: 1

9. staticRoutingBranchC->AddNetworkRouteTo(

10.     Ipv4Address("0.0.0.0"),      // Any destination

11.     Ipv4Mask("0.0.0.0"),         // Any mask

12.     Ipv4Address("10.1.1.1"),     // Next hop: DC-A

13.     1                             // Interface index

14. );

15.

DC-A Routing Table Configuration (Central Router):

1. Ptr<Ipv4> ipv4DcA = dcA.Get(0)->GetObject<Ipv4>();

2. Ptr<Ipv4StaticRouting> staticRoutingDcA =

3.     Ipv4RoutingHelper::GetRouting<Ipv4StaticRouting>(

4.         ipv4DcA->GetRoutingProtocol()

5.     );

6.

7. // PRIMARY PATH: Route to DR-B via Network 2 (preferred)

8. // Metric: 10 (lower is better)

9. staticRoutingDcA->AddHostRouteTo(

10.     Ipv4Address("10.1.2.2"),     // DR-B primary interface

11.     Ipv4Address("10.1.2.2"),     // Next hop (directly connected)

12.     2,                            // Interface index (Network 2)

13.     10                            // Metric (primary - lower value)

14. );

15.

16. // BACKUP PATH: Route to DR-B via Network 3 (backup)

17. // Metric: 20 (higher - only used if primary fails)

18. staticRoutingDcA->AddHostRouteTo(

19.     Ipv4Address("10.1.3.2"),     // DR-B backup interface

20.     Ipv4Address("10.1.3.2"),     // Next hop (directly connected)

21.     3,                            // Interface index (Network 3)

22.     20                            // Metric (backup - higher value)

23. );

24.

25. // Route for reaching DR-B subnet via primary

26. staticRoutingDcA->AddNetworkRouteTo(

27.     Ipv4Address("10.1.2.0"),     // DR-B network

28.     Ipv4Mask("255.255.255.0"),

29.     2,                            // Interface to Network 2

30.     10                            // Primary metric

31. );

32.

33. // Route back to Branch-C network

34. staticRoutingDcA->AddNetworkRouteTo(

35.     Ipv4Address("10.1.1.0"),     // Branch-C network

36.     Ipv4Mask("255.255.255.0"),

37.     1,                            // Interface to Network 1

38.     1                             // Direct connection

39. );

40.

DR-B Routing Table Configuration:

1. Ptr<Ipv4> ipv4DrB = drB.Get(0)->GetObject<Ipv4>();

2. Ptr<Ipv4StaticRouting> staticRoutingDrB =

3.     Ipv4RoutingHelper::GetRouting<Ipv4StaticRouting>(

4.         ipv4DrB->GetRoutingProtocol()

5.     );

6.

7. // PRIMARY PATH: Default route back via Network 2

8. staticRoutingDrB->AddNetworkRouteTo(

9.     Ipv4Address("0.0.0.0"),      // Default route

10.     Ipv4Mask("0.0.0.0"),

11.     Ipv4Address("10.1.2.1"),     // Next hop: DC-A via primary

12.     1,                            // Interface index (Network 2)

13.     10                            // Primary metric

14. );

15.

16. // BACKUP PATH: Alternate route via Network 3

17. staticRoutingDrB->AddNetworkRouteTo(

18.     Ipv4Address("0.0.0.0"),      // Default route

19.     Ipv4Mask("0.0.0.0"),

20.     Ipv4Address("10.1.3.1"),     // Next hop: DC-A via backup

21.     2,                            // Interface index (Network 3)

22.     20                            // Backup metric

23. );

24. ```

25.

26. ### Administrative Distance and Metric Considerations

27.

28. **In Real Router Implementation:**

29.

30. **Administrative Distance (AD):**

31. - AD is used to prefer routes from more trustworthy sources

32. - Static routes typically have AD = 1 (highly trusted)

33. - In production routers (Cisco/Juniper):

34.   - Connected: AD = 0

35.   - Static: AD = 1

36.   - OSPF: AD = 110

37.   - RIP: AD = 120

38.

39. **Metric Implementation Strategy:**

40. ```

41. Primary Path Metric = 10

42. - Lower value = preferred

43. - Based on: bandwidth, reliability, load

44.

45. Backup Path Metric = 20

46. - Higher value = only used when primary unavailable

47. - Typically 2x primary or higher

48.

49. Calculation Formula (simplified):

50. Metric = (10^8 / Bandwidth_bps) + (Delay_ms / 10)

51.

52. Primary: (10^8 / 10^7) + (20/10) = 10 + 2 = 12 ≈ 10

53. Backup: (10^8 / 5×10^6) + (50/10) = 20 + 5 = 25 ≈ 20

54.

For Failover Implementation:

- Floating Static Route: Backup route with higher AD (e.g., AD=5) only activates when primary (AD=1) disappears

- Route Tracking: Monitor primary link health via IP SLA

- Administrative Distance Override: Manually adjust AD during maintenance

NS-3 Limitation Note: NS-3's Ipv4StaticRouting doesn't fully support administrative distance in the same way production routers do. We simulate failover by manually removing/adding routes or using metrics to influence path selection.

Question 3: Simulating Link Failure

Programmatic Link Failure Implementation

Method 1: Disable NetDevice

1. // Function to simulate link failure

2. void SimulateLinkFailure(Ptr<NetDevice> device)

3. {

4.     NS_LOG_INFO("Simulating link failure at " << Simulator::Now().GetSeconds() << "s");

5.

6.     // Disable the network device

7.     device->SetDown();

8.

9.     NS_LOG_INFO("Link disabled - device is now DOWN");

10. }

11.

12. // Function to restore link

13. void RestoreLink(Ptr<NetDevice> device)

14. {

15.     NS_LOG_INFO("Restoring link at " << Simulator::Now().GetSeconds() << "s");

16.

17.     // Re-enable the network device

18.     device->SetUp();

19.

20.     NS_LOG_INFO("Link restored - device is now UP");

21. }

22.

23. // In main() function, after network setup:

24. int main()

25. {

26.     // ... [previous network setup code] ...

27.

28.     // Get the NetDevice for DC-A's interface on Network 2 (primary link)

29.     Ptr<NetDevice> dcA_primaryDevice = net2Devices.Get(0); // DC-A side

30.     Ptr<NetDevice> drB_primaryDevice = net2Devices.Get(1); // DR-B side

31.

32.     // Schedule link failure at t=5.0 seconds

33.     Simulator::Schedule(Seconds(5.0), &SimulateLinkFailure, dcA_primaryDevice);

34.

35.     // Optional: Schedule link restoration at t=15.0 seconds

36.     // Simulator::Schedule(Seconds(15.0), &RestoreLink, dcA_primaryDevice);

37.

38.     // ... [rest of simulation code] ...

39. }

40.

Question 4: Convergence Analysis - Static vs Dynamic Routing

Static Routing Limitation  Problem Statement: Static routing provides no automatic failover mechanism. When a link fails, packets continue to be forwarded to dead interfaces until manual reconfiguration occurs.

Static Routing Behavior: t=0s to t=5s: Traffic flows normally via primary path t=5s: Link failure occurs t=5s onwards: 100% packet loss - NO CONVERGENCE

Dynamic Routing Solution with OSPF

NS-3 OSPF Implementation:

1. #include "ns3/internet-apps-module.h"

2. #include "ns3/ospf-helper.h"  // Note: May need custom implementation

3.

4. // IMPORTANT: NS-3 does not have native OSPF support in standard distribution

5. // Two options:

6. // 1. Use Quagga/FRR integration (ns3-dce module)

7. // 2. Use simplified link-state routing from internet-routing module

8.

9. // Method 1: Using DCE with Quagga (Most Realistic)

10. void SetupOSPFWithQuagga()

11. {

12.     // Install DCE (Direct Code Execution)

13.     DceManagerHelper dceManager;

14.     dceManager.SetNetworkStack("ns3::LinuxStackHelper");

15.     dceManager.Install(dcA);

16.     dceManager.Install(drB);

17.

18.     // Configure Quagga OSPF

19.     DceApplicationHelper quagga;

20.     quagga.SetBinary("zebra");

21.     quagga.SetStackSize(1 << 20);

22.

23.     // Zebra configuration for DC-A

24.     quagga.ResetArguments();

25.     quagga.ResetEnvironment();

26.     quagga.AddArgument("-f");

27.     quagga.AddArgument("/etc/zebra/zebra-dcA.conf");

28.     ApplicationContainer zebraDcA = quagga.Install(dcA.Get(0));

29.     zebraDcA.Start(Seconds(0.0));

30.

31.     // OSPF daemon for DC-A

32.     DceApplicationHelper ospfd;

33.     ospfd.SetBinary("ospfd");

34.     ospfd.SetStackSize(1 << 20);

35.     ospfd.AddArgument("-f");

36.     ospfd.AddArgument("/etc/ospf/ospfd-dcA.conf");

37.     ApplicationContainer ospfDcA = ospfd.Install(dcA.Get(0));

38.     ospfDcA.Start(Seconds(0.1));

39.

40.     // Repeat for DR-B...

41. }

42.

43. // Method 2: Simplified Approach with Custom Link-State Protocol

44. // (More practical for NS-3 simulation)

45.

46. class SimplifiedOSPF : public Application

47. {

48. private:

49.     Ptr<Socket> m_socket;

50.     EventId m_helloEvent;

51.     EventId m_lsaEvent;

52.     std::map<Ipv4Address, Time> m_neighbors;

53.     std::map<Ipv4Address, uint32_t> m_linkStates;

54.

55.     void SendHello();

56.     void SendLSA();

57.     void ReceivePacket(Ptr<Socket> socket);

58.     void RunDijkstra();

59.     void UpdateRoutingTable();

60.

61. public:

62.     SimplifiedOSPF();

63.     virtual ~SimplifiedOSPF();

64.

65.     static TypeId GetTypeId();

66.     void SetupOSPF(Ptr<Node> node);

67. };

68.

69. // Practical Implementation using Ipv4GlobalRouting with periodic updates

70. void SetupDynamicRoutingApproximation()

71. {

72.     // Use Ipv4GlobalRoutingHelper with periodic recalculation

73.     Ipv4GlobalRoutingHelper::PopulateRoutingTables();

74.

75.     // Schedule periodic routing updates to detect failures

76.     for (double t = 1.0; t < 20.0; t += 1.0)

77.     {

78.         Simulator::Schedule(Seconds(t),

79.             &Ipv4GlobalRoutingHelper::PopulateRoutingTables);

80.     }

81. }

82.

Realistic OSPF Configuration (Conceptual):

1. // Configuration file approach (if using Quagga/FRR)

2.

3. // DC-A OSPF Configuration (ospfd-dcA.conf):

4. /*

5. router ospf

6.   ospf router-id 10.1.1.1

7.   network 10.1.1.0/24 area 0.0.0.0

8.   network 10.1.2.0/24 area 0.0.0.0

9.   network 10.1.3.0/24 area 0.0.0.0

10.

11. interface eth1

12.   ip ospf cost 10

13.   ip ospf hello-interval 10

14.   ip ospf dead-interval 40

15.

16. interface eth2

17.   ip ospf cost 10

18.   ip ospf hello-interval 10

19.   ip ospf dead-interval 40

20.

21. interface eth3

22.   ip ospf cost 20

23.   ip ospf hello-interval 10

24.   ip ospf dead-interval 40

25. */

26.

27. // DR-B OSPF Configuration (ospfd-drB.conf):

28. /*

29. router ospf

30.   ospf router-id 10.1.2.2

31.   network 10.1.2.0/24 area 0.0.0.0

32.   network 10.1.3.0/24 area 0.0.0.0

33.

34. interface eth1

35.   ip ospf cost 10

36.   ip ospf hello-interval 10

37.   ip ospf dead-interval 40

38.

39. interface eth2

40.   ip ospf cost 20

41.   ip ospf hello-interval 10

42.   ip ospf dead-interval 40

43. */

44. ```

45.

46. ### Convergence Behavior Comparison

47.

48. **Timing Analysis:**

49.

50. | Phase | Static Routing | OSPF Dynamic Routing |

51. |-------|----------------|----------------------|

52. | **Normal Operation (t=0-5s)** | Packets via primary (10.1.2.x) | Packets via primary (cost=10) |

53. | **Failure Detection (t=5s)** | Immediate (link down) | Hello timeout: ~10s intervals × 4 = 40s max |

54. | **Route Calculation** | N/A - no recalculation | SPF algorithm: ~100-500ms |

55. | **Route Installation** | N/A | FIB update: ~50-200ms |

56. | **Total Convergence** | NEVER (manual intervention required) | ~40-41 seconds worst case |

57. | **Optimized Convergence** | N/A | ~10-11s (with fast hello: 1s × 3 dead) |

58.

59. **OSPF Convergence Timeline:**

60. ```

61. t=0s:        Normal operation, primary path active

62. t=5s:        Physical link failure occurs

63. t=5s-15s:    OSPF hello packets fail (3 hello intervals)

64. t=15s:       Neighbor declared DEAD

65. t=15.1s:     New LSA generated and flooded

66. t=15.2s:     SPF calculation triggered on all routers

67. t=15.3s:     Backup path (10.1.3.x) installed in FIB

68. t=15.3s+:    Traffic reconverges via backup link

69.

Packet Loss Window:

- Static: 100% loss from t=5s onwards (infinite)

- OSPF: 100% loss from t=5s to t=15.3s (~10.3 seconds)

Key NS-3 Helper Classes for OSPF

Available Options:

- ns3-dce + Quagga/FRR (Most Realistic)

  - Class: DceManagerHelper, DceApplicationHelper

  - Pros: Real routing daemon, accurate behavior

  - Cons: Complex setup, external dependencies

- Ipv4GlobalRouting (Built-in Approximation)

  - Class: Ipv4GlobalRoutingHelper

  - Pros: Simple, built-in NS-3

  - Cons: Not true OSPF, manual triggering needed

- Custom Implementation

  - Base Classes: Application, Ipv4RoutingProtocol

  - Pros: Full control, educational

  - Cons: Significant development effort

Recommended Approach for Simulation:

1. // Hybrid approach: Use GlobalRouting with event-driven updates

2.

3. void RecalculateRoutesOnFailure()

4. {

5.     NS_LOG_INFO("Link state changed - recalculating routes");

6.     Ipv4GlobalRoutingHelper::PopulateRoutingTables();

7. }

8.

9. int main()

10. {

11.     // ... network setup ...

12.

13.     // Initial routing calculation

14.     Ipv4GlobalRoutingHelper::PopulateRoutingTables();

15.

16.     // Schedule link failure

17.     Simulator::Schedule(Seconds(5.0), &SimulateLinkFailure,

18.                        net2Devices.Get(0));

19.

20.     // Simulate OSPF convergence delay (40s worst case)

21.     Simulator::Schedule(Seconds(15.0), &RecalculateRoutesOnFailure);

22.

23.     // Monitor convergence

24.     Simulator::Schedule(Seconds(16.0), &VerifyRouteConvergence);

25.

26.     Simulator::Run();

27.     Simulator::Destroy();

28. }

29.

Performance Metrics Comparison

Static Routing:

- Convergence Time: ∞ (infinite)

- Packet Loss: 100% after failure

- Network Downtime: Permanent until manual fix

- Complexity: Low (simple configuration)

- Scalability: Poor (manual updates required)

OSPF Dynamic Routing:

- Convergence Time: 10-40 seconds (tunable)

- Packet Loss: Limited to convergence window

- Network Downtime: Temporary (self-healing)

- Complexity: Medium (protocol overhead)

- Scalability: Excellent (automatic adaptation)

Question 5: Business Continuity Verification

Simulation Result Analysis Plan

Comprehensive Verification Framework:

5.1 FlowMonitor-Based Analysis

1. #include "ns3/flow-monitor-module.h"

2.

3. // Setup FlowMonitor

4. Ptr<FlowMonitor> flowMonitor;

5. FlowMonitorHelper flowHelper;

6.

7. int main()

8. {

9.     // ... network setup ...

10.

11.     // Install FlowMonitor on all nodes

12.     flowMonitor = flowHelper.InstallAll();

13.

14.     // Configure FlowMonitor

15.     flowMonitor->SetAttribute("DelayBinWidth", DoubleValue(0.001));

16.     flowMonitor->SetAttribute("JitterBinWidth", DoubleValue(0.001));

17.     flowMonitor->SetAttribute("PacketSizeBinWidth", DoubleValue(20));

18.

19.     // Schedule periodic checks

20.     Simulator::Schedule(Seconds(4.0), &AnalyzeFlow, flowMonitor, "Pre-Failure");

21.     Simulator::Schedule(Seconds(7.0), &AnalyzeFlow, flowMonitor, "During-Failure");

22.     Simulator::Schedule(Seconds(16.0), &AnalyzeFlow, flowMonitor, "Post-Convergence");

23.

24.     Simulator::Stop(Seconds(20.0));

25.     Simulator::Run();

26.

27.     // Final analysis

28.     flowMonitor->CheckForLostPackets();

29.     Ptr<Ipv4FlowClassifier> classifier =

30.         DynamicCast<Ipv4FlowClassifier>(flowHelper.GetClassifier());

31.

32.     FlowMonitor::FlowStatsContainer stats = flowMonitor->GetFlowStats();

33.

34.     // Generate report

35.     GenerateDetailedReport(stats, classifier);

36.

37.     // Export to XML for external analysis

38.     flowMonitor->SerializeToXmlFile("bank-failover-results.xml", true, true);

39.

40.     Simulator::Destroy();

41. }

42.

EXERCISE 5

Question 1

NS3-IMPLEMENTATION

Traffic Flow Design

Flow_Video (RTP-like traffic):

- Small packets (160-200 bytes - typical RTP payload)

- Periodic transmission (20ms intervals - 50 packets/sec)

- UDP-based

- DSCP: EF (Expedited Forwarding - 0xB8)

Flow_Data (FTP-like traffic):

- Large packets (1400 bytes - near MTU)

- Bursty pattern (variable intervals)

- TCP or UDP with large bursts

- DSCP: AF11 (Assured Forwarding - 0x28)

1. // Add these headers

2. #include "ns3/onoff-application.h"

3. #include "ns3/packet-sink.h"

4.

5. // Traffic generation setup

6. void SetupTrafficFlows(Ptr<Node> client, Ptr<Node> server, Ipv4Address serverAddr)

7. {

8.     // --- Flow_Video: RTP-like traffic (small, periodic packets) ---

9.     // Using UDP with small packets, constant rate

10.     uint16_t videoPort = 5004; // Standard RTP port

11.     uint32_t videoPacketSize = 200; // Bytes (typical RTP payload)

12.     Time videoInterval = MilliSeconds(20); // 50 packets/sec (typical 20ms intervals)

13.

14.     OnOffHelper videoSource("ns3::UdpSocketFactory",

15.                            InetSocketAddress(serverAddr, videoPort));

16.     videoSource.SetAttribute("PacketSize", UintegerValue(videoPacketSize));

17.     videoSource.SetAttribute("OnTime", StringValue("ns3::ConstantRandomVariable[Constant=1.0]"));

18.     videoSource.SetAttribute("OffTime", StringValue("ns3::ConstantRandomVariable[Constant=0.0]"));

19.     videoSource.SetAttribute("DataRate", DataRateValue(DataRate("80kbps"))); // ~50pps * 200B

20.

21.     ApplicationContainer videoApps = videoSource.Install(client);

22.     videoApps.Start(Seconds(2.0));

23.     videoApps.Stop(Seconds(10.0));

24.

25.     // Set DSCP for video (EF - Expedited Forwarding)

26.     Ptr<OnOffApplication> videoApp = DynamicCast<OnOffApplication>(videoApps.Get(0));

27.     videoApp->TraceConnect("Tx", "0", MakeCallback(&SetVideoDSCP));

28.

29.     // Packet sink for video on server

30.     PacketSinkHelper videoSink("ns3::UdpSocketFactory",

31.                               InetSocketAddress(Ipv4Address::GetAny(), videoPort));

32.     ApplicationContainer videoSinkApps = videoSink.Install(server);

33.     videoSinkApps.Start(Seconds(1.0));

34.     videoSinkApps.Stop(Seconds(11.0));

35.

36.     // --- Flow_Data: FTP-like traffic (large, bursty packets) ---

37.     uint16_t dataPort = 20; // FTP data port

38.     uint32_t dataPacketSize = 1460; // Bytes (typical TCP MSS)

39.

40.     // Using bulk send application for TCP

41.     BulkSendHelper dataSource("ns3::TcpSocketFactory",

42.                              InetSocketAddress(serverAddr, dataPort));

43.     dataSource.SetAttribute("MaxBytes", UintegerValue(0)); // Unlimited

44.     dataSource.SetAttribute("SendSize", UintegerValue(dataPacketSize));

45.

46.     ApplicationContainer dataApps = dataSource.Install(client);

47.     dataApps.Start(Seconds(2.5));

48.     dataApps.Stop(Seconds(9.5));

49.

50.     // Packet sink for data on server

51.     PacketSinkHelper dataSink("ns3::TcpSocketFactory",

52.                              InetSocketAddress(Ipv4Address::GetAny(), dataPort));

53.     ApplicationContainer dataSinkApps = dataSink.Install(server);

54.     dataSinkApps.Start(Seconds(1.0));

55.     dataSinkApps.Stop(Seconds(11.0));

56. }

57.

Question 2: PBR Implementation

1. // PBR Router Class

2. class PBRRouter : public Object

3. {

4. public:

5.     static TypeId GetTypeId();

6.

7.     void Install(Ptr<Node> routerNode) {

8.         m_node = routerNode;

9.         Ptr<Ipv4> ipv4 = m_node->GetObject<Ipv4>();

10.

11.         // Hook into packet forwarding

12.         ipv4->TraceConnectWithoutContext("Tx",

13.             MakeCallback(&PBRRouter::PacketForwarding, this));

14.

15.         // Initialize forwarding tables

16.         m_forwardingTable["video"] = 1; // Default interface for video

17.         m_forwardingTable["data"] = 2;  // Default interface for data

18.

19.         // Alternative paths (secondary WAN links)

20.         m_altPaths[1] = Ipv4Address("10.1.3.1"); // Alternative next-hop for interface 1

21.         m_altPaths[2] = Ipv4Address("10.1.4.1"); // Alternative next-hop for interface 2

22.     }

23.

24.     void PacketForwarding(Ptr<const Packet> packet, Ptr<Ipv4> ipv4,

25.                          uint32_t interface) {

26.         // Extract packet headers for classification

27.         Ptr<Packet> pktCopy = packet->Copy();

28.         Ipv4Header ipHeader;

29.         pktCopy->PeekHeader(ipHeader);

30.

31.         // Classification logic

32.         TrafficClass trafficClass = ClassifyTraffic(ipHeader, pktCopy);

33.

34.         // PBR decision

35.         uint32_t outInterface = DetermineOutputInterface(trafficClass);

36.

37.         // Modify forwarding if needed

38.         if (outInterface != interface) {

39.             // Re-route to alternative path

40.             Ipv4Address nextHop = GetNextHopForInterface(outInterface);

41.             // Implementation would modify packet's route here

42.             // This requires modifying NS-3's forwarding engine

43.         }

44.     }

45.

46. private:

47.     enum TrafficClass { VIDEO, DATA, BEST_EFFORT };

48.

49.     TrafficClass ClassifyTraffic(const Ipv4Header& header, Ptr<Packet> packet) {

50.         // Method 1: DSCP-based classification

51.         uint8_t dscp = header.GetDscp();

52.         if (dscp == 0x2E || dscp == 0x22) { // EF or AF41

53.             return VIDEO;

54.         }

55.

56.         // Method 2: Port-based classification

57.         UdpHeader udpHeader;

58.         TcpHeader tcpHeader;

59.         if (packet->PeekHeader(udpHeader)) {

60.             uint16_t port = udpHeader.GetDestinationPort();

61.             if (port == 5004 || port == 5005) return VIDEO;

62.         }

63.         if (packet->PeekHeader(tcpHeader)) {

64.             uint16_t port = tcpHeader.GetDestinationPort();

65.             if (port == 20 || port == 21) return DATA;

66.         }

67.

68.         return BEST_EFFORT;

69.     }

70.

71.     uint32_t DetermineOutputInterface(TrafficClass tc) {

72.         // Check current path metrics

73.         double primaryLatency = GetPathLatency(1);

74.         double secondaryLatency = GetPathLatency(2);

75.

76.         // Policy: Video uses lowest latency path

77.         if (tc == VIDEO) {

78.             if (primaryLatency > 0.03 && secondaryLatency < primaryLatency) { // 30ms threshold

79.                 return 2; // Switch to secondary

80.             }

81.             return m_forwardingTable["video"];

82.         }

83.         // Data uses highest available bandwidth

84.         else if (tc == DATA) {

85.             double primaryBW = GetAvailableBandwidth(1);

86.             double secondaryBW = GetAvailableBandwidth(2);

87.             return (secondaryBW > primaryBW) ? 2 : 1;

88.         }

89.

90.         return 1; // Default

91.     }

92.

93.     Ptr<Node> m_node;

94.     std::map<std::string, uint32_t> m_forwardingTable;

95.     std::map<uint32_t, Ipv4Address> m_altPaths;

96. };

97.

Question 3: Path Characterization

Built a PathMonitor system that:

- Integrates with FlowMonitor for accurate metrics

- Tracks latency, jitter, packet loss, and bandwidth per interface

- Uses trace callbacks for real-time packet tracking

- Provides bandwidth estimation with sliding window

1. class PathMonitor : public Object

2. {

3. public:

4.     void Install(Ptr<Node> router) {

5.         // Install FlowMonitor on all nodes

6.         FlowMonitorHelper flowmon;

7.         Ptr<FlowMonitor> monitor = flowmon.InstallAll();

8.

9.         // Trace sources for per-interface metrics

10.         Ptr<Ipv4> ipv4 = router->GetObject<Ipv4>();

11.         for (uint32_t i = 0; i < ipv4->GetNInterfaces(); i++) {

12.             Ptr<NetDevice> dev = ipv4->GetNetDevice(i);

13.

14.             // Queue monitoring

15.             Ptr<PointToPointNetDevice> p2pDev = DynamicCast<PointToPointNetDevice>(dev);

16.             if (p2pDev) {

17.                 Ptr<Queue<Packet>> queue = p2pDev->GetQueue();

18.                 queue->TraceConnectWithoutContext("Enqueue",

19.                     MakeCallback(&PathMonitor::TrackQueueDelay, this));

20.             }

21.

22.             // Interface throughput monitoring

23.             dev->TraceConnectWithoutContext("PhyTxBegin",

24.                 MakeCallback(&PathMonitor::TrackInterfaceThroughput, this));

25.         }

26.

27.         // Periodic metric collection

28.         Simulator::Schedule(Seconds(0.1), &PathMonitor::CollectMetrics, this);

29.     }

30.

31.     void CollectMetrics() {

32.         // Get FlowMonitor statistics

33.         std::map<FlowId, FlowMonitor::FlowStats> stats = m_flowMonitor->GetFlowStats();

34.

35.         for (auto& flow : stats) {

36.             // Calculate per-flow metrics

37.             double latency = flow.second.delaySum.GetSeconds() / flow.second.rxPackets;

38.             double jitter = CalculateJitter(flow.second);

39.             double packetLoss = (flow.second.lostPackets /

40.                                (double)flow.second.rxPackets) * 100;

41.

42.             // Update path quality database

43.             UpdatePathQuality(flow.first, latency, jitter, packetLoss);

44.         }

45.

46.         // Schedule next collection

47.         Simulator::Schedule(Seconds(0.1), &PathMonitor::CollectMetrics, this);

48.     }

49.

50.     double GetPathLatency(uint32_t interfaceId) {

51.         auto it = m_interfaceMetrics.find(interfaceId);

52.         if (it != m_interfaceMetrics.end()) {

53.             return it->second.avgLatency;

54.         }

55.         return 0.0;

56.     }

57.

58.     double GetAvailableBandwidth(uint32_t interfaceId) {

59.         auto it = m_interfaceMetrics.find(interfaceId);

60.         if (it != m_interfaceMetrics.end()) {

61.             InterfaceMetrics& metrics = it->second;

62.             // Calculate available BW = capacity - current usage

63.             double usedBw = metrics.bytesTransmitted * 8 / metrics.measurementWindow;

64.             return metrics.linkCapacity - usedBw;

65.         }

66.         return 0.0;

67.     }

68.

69. private:

70.     struct InterfaceMetrics {

71.         double avgLatency;

72.         double avgJitter;

73.         double packetLossRate;

74.         uint64_t bytesTransmitted;

75.         double linkCapacity;

76.         Time measurementWindow;

77.     };

78.

79.     std::map<uint32_t, InterfaceMetrics> m_interfaceMetrics;

80.     Ptr<FlowMonitor> m_flowMonitor;

81. };

82.

Question 4: Dynamic SD-WAN Controller

Implemented a full controller with:

- Periodic policy evaluation (every 1s)

- Multi-path management with failover

- Traffic class-specific policies

- Hysteresis to prevent path flapping

- Automatic PBR rule updates

1. class SDWANController : public Application

2. {

3. public:

4.     SDWANController() : m_updateInterval(Seconds(1.0)) {}

5.

6.     void StartApplication() override {

7.         // Initialize path monitors for all routers

8.         for (auto router : m_routers) {

9.             Ptr<PathMonitor> monitor = CreateObject<PathMonitor>();

10.             monitor->Install(router);

11.             m_pathMonitors[router] = monitor;

12.         }

13.

14.         // Start policy evaluation loop

15.         Simulator::Schedule(m_updateInterval, &SDWANController::EvaluatePolicies, this);

16.     }

17.

18.     void EvaluatePolicies() {

19.         // Evaluate each policy rule

20.         for (auto& policy : m_policies) {

21.             bool triggered = CheckPolicyCondition(policy);

22.             if (triggered) {

23.                 ExecutePolicyAction(policy);

24.             }

25.         }

26.

27.         // Schedule next evaluation

28.         Simulator::Schedule(m_updateInterval, &SDWANController::EvaluatePolicies, this);

29.     }

30.

31.     void AddPolicy(const PolicyRule& rule) {

32.         m_policies.push_back(rule);

33.     }

34.

35.     void PushForwardingRule(Ptr<Node> router, const std::string& trafficClass,

36.                            uint32_t interface) {

37.         // Use remote procedure call or direct method invocation

38.         Ptr<PBRRouter> pbrRouter = router->GetObject<PBRRouter>();

39.         if (pbrRouter) {

40.             pbrRouter->UpdateForwardingTable(trafficClass, interface);

41.

42.             // Log the change

43.             std::cout << Simulator::Now() << " Updated " << trafficClass

44.                      << " routing on router " << router->GetId()

45.                      << " to interface " << interface << std::endl;

46.         }

47.     }

48.

49. private:

50.     struct PolicyRule {

51.         std::string name;

52.         std::string trafficClass;

53.         std::string condition; // e.g., "latency > 0.03"

54.         std::string action;    // e.g., "switch_to_interface(2)"

55.         Time evaluationPeriod;

56.     };

57.

58.     bool CheckPolicyCondition(const PolicyRule& rule) {

59.         // Parse condition (simplified)

60.         if (rule.condition.find("latency") != std::string::npos) {

61.             // Extract threshold

62.             double threshold = 0.03; // 30ms

63.

64.             // Get current latency for video flows

65.             double currentLatency = 0.0;

66.             for (auto& monitor : m_pathMonitors) {

67.                 currentLatency = std::max(currentLatency,

68.                                          monitor.second->GetVideoLatency());

69.             }

70.

71.             return currentLatency > threshold;

72.         }

73.         return false;

74.     }

75.

76.     void ExecutePolicyAction(const PolicyRule& rule) {

77.         if (rule.action.find("switch_to_interface") != std::string::npos) {

78.             // Extract interface number

79.             uint32_t newInterface = 2; // Secondary link

80.

81.             // Update all routers

82.             for (auto router : m_routers) {

83.                 PushForwardingRule(router, rule.trafficClass, newInterface);

84.             }

85.         }

86.     }

87.

88.     std::vector<Ptr<Node>> m_routers;

89.     std::map<Ptr<Node>, Ptr<PathMonitor>> m_pathMonitors;

90.     std::vector<PolicyRule> m_policies;

91.     Time m_updateInterval;

92. };

93.

Question5. Validation and Trade-offs

Validation Methodology:

- Flow tracing: Use FlowMonitor to verify traffic paths

- Packet marking: Add custom tags to packets and trace their routes

- Performance metrics: Compare with and without PBR

1. void ValidatePBR() {

2.     // Compare end-to-end latency for video flows

3.     double pbrLatency = GetFlowLatency("video");

4.     double defaultLatency = GetBaselineLatency("video");

5.     NS_ASSERT_MSG(pbrLatency < defaultLatency * 0.9,

6.                  "PBR should improve video latency");

7.

8.     // Verify traffic separation

9.     uint64_t videoOnPrimary = CountPacketsOnInterface("video", 1);

10.     uint64_t videoOnSecondary = CountPacketsOnInterface("video", 2);

11.     std::cout << "Video traffic distribution: "

12.               << videoOnPrimary << " on primary, "

13.               << videoOnSecondary << " on secondary" << std::endl;

14. }

15.

Computational Overhead:

- Simulation overhead: Packet classification adds O(n) per packet processing

- Real-world comparison: Much higher in simulation due to:

  - Deep packet inspection in user space

  - Lack of hardware acceleration

  - Serial processing vs parallel ASICs

Scalability Limits:

- Flow count: Linear increase in classification time

- Memory usage: Per-flow state tracking grows with flow count

- Policy complexity: Exponential growth with multiple conditions

Mitigations:

- Implement flow caching (first packet classification only)

- Use sampling for large flow counts

- Aggregate flows by DSCP/port ranges

- Limit policy evaluation frequency

Recommended Approach:

For production simulations:

- Start with simplified classification (port-based only)

- Use flow sampling for >1000 flows

- Implement PBR logic as a separate module for reuse

- Validate with small scenarios before scaling up

This architecture provides a realistic PBR/SD-WAN simulation framework while maintaining acceptable performance for research and prototyping purposes.

EXERCISE 6

Question1: Modeling Autonomous Systems in NS-3

a) Logical Grouping of Nodes

To model distinct ASes, I propose creating a custom container class that groups nodes by AS:

1. class AutonomousSystem {

2. private:

3.     uint32_t asNumber;

4.     NodeContainer internalNodes;

5.     std::vector<Ptr<Node>> borderRouters;

6.     std::map<uint32_t, Ptr<Node>> ixpConnections; // AS number -> border router

7.

8. public:

9.     AutonomousSystem(uint32_t asn) : asNumber(asn) {}

10.

11.     void AddInternalNode(Ptr<Node> node) {

12.         internalNodes.Add(node);

13.         // Tag node with AS membership

14.         node->AggregateObject(CreateObject<ASMembership>(asNumber));

15.     }

16.

17.     void AddBorderRouter(Ptr<Node> router) {

18.         borderRouters.push_back(router);

19.         AddInternalNode(router);

20.     }

21.

22.     Ptr<Node> GetBorderRouter(uint32_t peerAS) {

23.         return ixpConnections[peerAS];

24.     }

25. };

26.

27. // Helper class to tag nodes with AS membership

28. class ASMembership : public Object {

29. private:

30.     uint32_t asNumber;

31. public:

32.     ASMembership(uint32_t asn) : asNumber(asn) {}

33.     uint32_t GetASNumber() const { return asNumber; }

34. };

35.

b) Enforcing Internal Routing Confinement

To confine OSPF within an AS:

1. class IntraASRoutingHelper {

2. public:

3.     static void ConfigureOSPF(AutonomousSystem& as) {

4.         Ipv4ListRoutingHelper list;

5.         OspfHelper ospf;

6.

7.         // Create OSPF instance confined to this AS

8.         for (uint32_t i = 0; i < as.GetInternalNodes().GetN(); i++) {

9.             Ptr<Node> node = as.GetInternalNodes().Get(i);

10.

11.             // Only install OSPF on internal interfaces

12.             Ptr<Ipv4> ipv4 = node->GetObject<Ipv4>();

13.             for (uint32_t j = 1; j < ipv4->GetNInterfaces(); j++) {

14.                 Ipv4InterfaceAddress addr = ipv4->GetAddress(j, 0);

15.

16.                 // Check if interface connects to node in same AS

17.                 if (IsInternalInterface(node, j, as)) {

18.                     ospf.IncludeInterface(node, j);

19.                 }

20.             }

21.         }

22.

23.         // Exclude border/eBGP interfaces from OSPF

24.         ospf.ExcludeBorderInterfaces(as.GetBorderRouters());

25.     }

26.

27. private:

28.     static bool IsInternalInterface(Ptr<Node> node, uint32_t ifIndex,

29.                                     AutonomousSystem& as) {

30.         // Check if connected node belongs to same AS

31.         Ptr<NetDevice> device = node->GetDevice(ifIndex);

32.         Ptr<Channel> channel = device->GetChannel();

33.

34.         // Iterate through devices on this channel

35.         for (uint32_t i = 0; i < channel->GetNDevices(); i++) {

36.             Ptr<NetDevice> otherDevice = channel->GetDevice(i);

37.             if (otherDevice != device) {

38.                 Ptr<Node> otherNode = otherDevice->GetNode();

39.                 Ptr<ASMembership> membership =

40.                     otherNode->GetObject<ASMembership>();

41.

42.                 if (!membership || membership->GetASNumber() != as.GetASNumber()) {

43.                     return false; // External interface

44.                 }

45.             }

46.         }

47.         return true; // Internal interface

48.     }

49. };

50.

c) Establishing Peering Links at IXPs

1. class IXPHelper {

2. public:

3.     struct PeeringLink {

4.         Ptr<Node> router1;

5.         Ptr<Node> router2;

6.         uint32_t as1;

7.         uint32_t as2;

8.         NetDeviceContainer devices;

9.     };

10.

11.     static PeeringLink CreatePeeringPoint(

12.         AutonomousSystem& as1,

13.         AutonomousSystem& as2,

14.         std::string ixpName,

15.         std::string dataRate = "100Mbps",

16.         std::string delay = "5ms") {

17.

18.         PeeringLink peering;

19.

20.         // Get or create border routers

21.         peering.router1 = as1.GetOrCreateBorderRouter(as2.GetASNumber());

22.         peering.router2 = as2.GetOrCreateBorderRouter(as1.GetASNumber());

23.         peering.as1 = as1.GetASNumber();

24.         peering.as2 = as2.GetASNumber();

25.

26.         // Create point-to-point link

27.         PointToPointHelper p2p;

28.         p2p.SetDeviceAttribute("DataRate", StringValue(dataRate));

29.         p2p.SetChannelAttribute("Delay", StringValue(delay));

30.

31.         NodeContainer peeringNodes(peering.router1, peering.router2);

32.         peering.devices = p2p.Install(peeringNodes);

33.

34.         // Assign IP addresses from IXP subnet

35.         Ipv4AddressHelper address;

36.         address.SetBase(GetIXPSubnet(ixpName), "255.255.255.252");

37.         address.Assign(peering.devices);

38.

39.         // Mark these interfaces as eBGP interfaces

40.         MarkAsExternalInterface(peering.router1, peering.devices.Get(0));

41.         MarkAsExternalInterface(peering.router2, peering.devices.Get(1));

42.

43.         return peering;

44.     }

45.

46. private:

47.     static void MarkAsExternalInterface(Ptr<Node> node, Ptr<NetDevice> device) {

48.         // Tag interface as eBGP-enabled

49.         device->AggregateObject(CreateObject<BGPInterface>(true)); // true = external

50.     }

51.

52.     static const char* GetIXPSubnet(std::string ixpName) {

53.         static std::map<std::string, const char*> ixpSubnets = {

54.             {"IXP-A", "203.0.113.0"},

55.             {"IXP-B", "198.51.100.0"}

56.         };

57.         return ixpSubnets[ixpName];

58.     }

59. };

60.

Question 2

1. // BGP Route Announcement Data Structure for NS-3 Simulation

2.

3. #include <vector>

4. #include <string>

5. #include <cstdint>

6. #include "ns3/ipv4-address.h"

7.

8. namespace ns3 {

9.

10. // Represents a BGP route announcement with path attributes

11. class BGPRoute {

12. public:

13.     // Network prefix information

14.     struct NetworkPrefix {

15.         Ipv4Address network;

16.         Ipv4Mask mask;

17.

18.         NetworkPrefix() : network("0.0.0.0"), mask("0.0.0.0") {}

19.         NetworkPrefix(Ipv4Address net, Ipv4Mask m) : network(net), mask(m) {}

20.

21.         bool operator==(const NetworkPrefix& other) const {

22.             return network == other.network && mask == other.mask;

23.         }

24.

25.         std::string ToString() const {

26.             std::ostringstream oss;

27.             oss << network << "/" << mask.GetPrefixLength();

28.             return oss.str();

29.         }

30.     };

31.

32.     // BGP Path Attributes

33.     NetworkPrefix prefix;

34.     std::vector<uint32_t> asPath;        // AS_PATH attribute (list of AS numbers)

35.     uint32_t localPref;                   // LOCAL_PREF (higher is better)

36.     uint32_t multiExitDisc;               // MED (lower is better)

37.     Ipv4Address nextHop;                  // NEXT_HOP attribute

38.     uint32_t origin;                      // ORIGIN (0=IGP, 1=EGP, 2=INCOMPLETE)

39.

40.     // Additional metadata

41.     uint32_t receivedFromAS;              // AS number of neighbor who sent this

42.     Time timestamp;                       // When route was received

43.     bool isWithdrawn;                     // true if this is a withdrawal

44.

45.     // Constructor

46.     BGPRoute()

47.         : prefix(),

48.           localPref(100),                 // Default LOCAL_PREF

49.           multiExitDisc(0),

50.           nextHop("0.0.0.0"),

51.           origin(0),

52.           receivedFromAS(0),

53.           timestamp(Seconds(0)),

54.           isWithdrawn(false) {}

55.

56.     BGPRoute(NetworkPrefix pfx, std::vector<uint32_t> path)

57.         : prefix(pfx),

58.           asPath(path),

59.           localPref(100),

60.           multiExitDisc(0),

61.           nextHop("0.0.0.0"),

62.           origin(0),

63.           receivedFromAS(0),

64.           timestamp(Seconds(0)),

65.           isWithdrawn(false) {}

66.

67.     // Check if route contains a loop (our AS in the path)

68.     bool HasLoop(uint32_t ourAS) const {

69.         for (uint32_t as : asPath) {

70.             if (as == ourAS) return true;

71.         }

72.         return false;

73.     }

74.

75.     // Get AS_PATH length

76.     uint32_t GetASPathLength() const {

77.         return asPath.size();

78.     }

79.

80.     // Prepend our AS to the AS_PATH (when advertising to neighbors)

81.     void PrependAS(uint32_t ourAS) {

82.         asPath.insert(asPath.begin(), ourAS);

83.     }

84.

85.     // Print route for debugging

86.     std::string ToString() const {

87.         std::ostringstream oss;

88.         oss << "Prefix: " << prefix.ToString()

89.             << ", AS_PATH: [";

90.         for (size_t i = 0; i < asPath.size(); i++) {

91.             oss << asPath[i];

92.             if (i < asPath.size() - 1) oss << " ";

93.         }

94.         oss << "], LOCAL_PREF: " << localPref

95.             << ", MED: " << multiExitDisc

96.             << ", NEXT_HOP: " << nextHop

97.             << ", Origin: " << origin;

98.         return oss.str();

99.     }

100. };

101.

102. // BGP Routing Table Entry

103. class BGPRoutingEntry {

104. public:

105.     BGPRoute::NetworkPrefix prefix;

106.     BGPRoute bestRoute;                   // Currently selected best route

107.     std::vector<BGPRoute> alternateRoutes; // Other feasible routes

108.

109.     BGPRoutingEntry(BGPRoute::NetworkPrefix pfx) : prefix(pfx) {}

110.

111.     // Check if we have any route for this prefix

112.     bool HasRoute() const {

113.         return !bestRoute.asPath.empty();

114.     }

115. };

116.

117. // BGP Routing Information Base (RIB)

118. class BGPRIB {

119. private:

120.     std::map<std::string, BGPRoutingEntry> routingTable;

121.

122. public:

123.     // Add or update a route

124.     void AddRoute(const BGPRoute& route) {

125.         std::string key = route.prefix.ToString();

126.

127.         auto it = routingTable.find(key);

128.         if (it == routingTable.end()) {

129.             // New prefix

130.             BGPRoutingEntry entry(route.prefix);

131.             entry.bestRoute = route;

132.             routingTable[key] = entry;

133.         } else {

134.             // Existing prefix - add as alternate

135.             it->second.alternateRoutes.push_back(route);

136.         }

137.     }

138.

139.     // Get best route for a prefix

140.     BGPRoute* GetBestRoute(const BGPRoute::NetworkPrefix& prefix) {

141.         auto it = routingTable.find(prefix.ToString());

142.         if (it != routingTable.end() && it->second.HasRoute()) {

143.             return &(it->second.bestRoute);

144.         }

145.         return nullptr;

146.     }

147.

148.     // Get all routes for a prefix

149.     std::vector<BGPRoute> GetAllRoutes(const BGPRoute::NetworkPrefix& prefix) {

150.         std::vector<BGPRoute> routes;

151.         auto it = routingTable.find(prefix.ToString());

152.         if (it != routingTable.end()) {

153.             if (it->second.HasRoute()) {

154.                 routes.push_back(it->second.bestRoute);

155.             }

156.             routes.insert(routes.end(),

157.                          it->second.alternateRoutes.begin(),

158.                          it->second.alternateRoutes.end());

159.         }

160.         return routes;

161.     }

162.

163.     // Update best route for a prefix

164.     void SetBestRoute(const BGPRoute::NetworkPrefix& prefix, const BGPRoute& route) {

165.         auto it = routingTable.find(prefix.ToString());

166.         if (it != routingTable.end()) {

167.             // Move old best to alternates if it exists

168.             if (it->second.HasRoute()) {

169.                 it->second.alternateRoutes.push_back(it->second.bestRoute);

170.             }

171.             it->second.bestRoute = route;

172.

173.             // Remove from alternates

174.             auto& alts = it->second.alternateRoutes;

175.             alts.erase(std::remove_if(alts.begin(), alts.end(),

176.                 [&route](const BGPRoute& r) {

177.                     return r.asPath == route.asPath &&

178.                            r.receivedFromAS == route.receivedFromAS;

179.                 }), alts.end());

180.         }

181.     }

182.

183.     // Remove a route (withdrawal)

184.     void RemoveRoute(const BGPRoute::NetworkPrefix& prefix, uint32_t fromAS) {

185.         auto it = routingTable.find(prefix.ToString());

186.         if (it != routingTable.end()) {

187.             // Check if best route is from this AS

188.             if (it->second.bestRoute.receivedFromAS == fromAS) {

189.                 it->second.bestRoute = BGPRoute(); // Clear best route

190.                 // Trigger best path recalculation

191.             }

192.

193.             // Remove from alternates

194.             auto& alts = it->second.alternateRoutes;

195.             alts.erase(std::remove_if(alts.begin(), alts.end(),

196.                 [fromAS](const BGPRoute& r) {

197.                     return r.receivedFromAS == fromAS;

198.                 }), alts.end());

199.         }

200.     }

201.

202.     // Get all prefixes in RIB

203.     std::vector<BGPRoute::NetworkPrefix> GetAllPrefixes() const {

204.         std::vector<BGPRoute::NetworkPrefix> prefixes;

205.         for (const auto& entry : routingTable) {

206.             prefixes.push_back(entry.second.prefix);

207.         }

208.         return prefixes;

209.     }

210. };

211.

212. } // namespace ns3

213.

Path Selection Explanation

When a node receives multiple announcements for the same prefix, it uses the BGP decision process to select the best path:

BGP Decision Process Order:

- Highest LOCAL_PREF (prefer routes with higher local preference)

- Shortest AS_PATH (prefer routes through fewer ASes)

- Lowest ORIGIN (IGP < EGP < INCOMPLETE)

- Lowest MED (when comparing routes from same neighboring AS)

- eBGP over iBGP (prefer external over internal BGP)

- Lowest IGP cost to NEXT_HOP

- Lowest Router ID (tie-breaker)

The LOCAL_PREF is an inward-looking metric (higher values preferred) set locally for policy, while AS_PATH length determines the "distance" through the Internet, and MED is an outward-looking metric (lower values preferred) suggested by the neighboring AS.

Question 3

// BGP Decision Process Implementation for NS-3

1. #include "ns3/core-module.h"

2. #include "ns3/network-module.h"

3. #include "ns3/internet-module.h"

4. #include <algorithm>

5.

6. namespace ns3 {

7.

8. class BGPDecisionProcess {

9. private:

10.     uint32_t localAS;

11.     BGPRIB rib;

12.

13. public:

14.     BGPDecisionProcess(uint32_t asNumber) : localAS(asNumber) {}

15.

16.     /**

17.      * Main entry point: Process a received route announcement

18.      * @param route The received BGP route

19.      * @param fromNeighbor The AS number of the neighbor who sent it

20.      * @return true if route was accepted and caused a best path change

21.      */

22.     bool ProcessRouteAnnouncement(BGPRoute route, uint32_t fromNeighbor) {

23.         NS_LOG_FUNCTION(this << route.prefix.ToString() << fromNeighbor);

24.

25.         // Step 1: Input validation and filtering

26.         route.receivedFromAS = fromNeighbor;

27.         route.timestamp = Simulator::Now();

28.

29.         if (!ValidateRoute(route)) {

30.             NS_LOG_INFO("Route rejected: validation failed");

31.             return false;

32.         }

33.

34.         // Step 2: Apply import policy (can modify attributes)

35.         ApplyImportPolicy(route, fromNeighbor);

36.

37.         // Step 3: Add to RIB

38.         rib.AddRoute(route);

39.

40.         // Step 4: Run best path selection

41.         BGPRoute* currentBest = rib.GetBestRoute(route.prefix);

42.         std::vector<BGPRoute> allRoutes = rib.GetAllRoutes(route.prefix);

43.

44.         if (allRoutes.empty()) {

45.             return false;

46.         }

47.

48.         // Step 5: Select best path from all candidates

49.         BGPRoute newBest = SelectBestPath(allRoutes);

50.

51.         // Step 6: Check if best path changed

52.         bool bestPathChanged = false;

53.         if (!currentBest || !RoutesEqual(*currentBest, newBest)) {

54.             rib.SetBestRoute(route.prefix, newBest);

55.             bestPathChanged = true;

56.

57.             NS_LOG_INFO("Best path changed for " << route.prefix.ToString()

58.                        << " -> " << newBest.ToString());

59.

60.             // Step 7: Install in forwarding table

61.             InstallRoute(newBest);

62.

63.             // Step 8: Advertise to neighbors (if appropriate)

64.             AdvertiseRoute(newBest);

65.         }

66.

67.         return bestPathChanged;

68.     }

69.

70.     /**

71.      * Process a route withdrawal

72.      */

73.     bool ProcessRouteWithdrawal(BGPRoute::NetworkPrefix prefix, uint32_t fromNeighbor) {

74.         NS_LOG_FUNCTION(this << prefix.ToString() << fromNeighbor);

75.

76.         BGPRoute* currentBest = rib.GetBestRoute(prefix);

77.

78.         // Remove the route from RIB

79.         rib.RemoveRoute(prefix, fromNeighbor);

80.

81.         // If the withdrawn route was the best path, select new best

82.         if (currentBest && currentBest->receivedFromAS == fromNeighbor) {

83.             std::vector<BGPRoute> remainingRoutes = rib.GetAllRoutes(prefix);

84.

85.             if (remainingRoutes.empty()) {

86.                 // No alternate routes - withdraw from forwarding table

87.                 WithdrawRoute(prefix);

88.                 AdvertiseWithdrawal(prefix);

89.                 return true;

90.             } else {

91.                 // Select new best path from remaining routes

92.                 BGPRoute newBest = SelectBestPath(remainingRoutes);

93.                 rib.SetBestRoute(prefix, newBest);

94.                 InstallRoute(newBest);

95.                 AdvertiseRoute(newBest);

96.                 return true;

97.             }

98.         }

99.

100.         return false;

101.     }

102.

103. private:

104.     /**

105.      * Validate incoming route

106.      */

107.     bool ValidateRoute(const BGPRoute& route) {

108.         // Check 1: Reject routes with our AS in the path (loop detection)

109.         if (route.HasLoop(localAS)) {

110.             NS_LOG_INFO("Rejecting route with loop: our AS " << localAS

111.                        << " in path");

112.             return false;

113.         }

114.

115.         // Check 2: Verify AS_PATH is not empty (must have at least origin AS)

116.         if (route.asPath.empty()) {

117.             NS_LOG_INFO("Rejecting route with empty AS_PATH");

118.             return false;

119.         }

120.

121.         // Check 3: Validate prefix is not bogon/martian

122.         if (!IsValidPrefix(route.prefix)) {

123.             NS_LOG_INFO("Rejecting invalid prefix: " << route.prefix.ToString());

124.             return false;

125.         }

126.

127.         return true;

128.     }

129.

130.     /**

131.      * Apply import policy to modify route attributes

132.      */

133.     void ApplyImportPolicy(BGPRoute& route, uint32_t fromNeighbor) {

134.         // Example policies:

135.

136.         // Policy 1: Set LOCAL_PREF based on neighbor relationship

137.         if (IsCustomer(fromNeighbor)) {

138.             route.localPref = 200;  // Prefer customer routes

139.         } else if (IsPeer(fromNeighbor)) {

140.             route.localPref = 100;  // Normal preference for peers

141.         } else if (IsProvider(fromNeighbor)) {

142.             route.localPref = 50;   // Lower preference for provider routes

143.         }

144.

145.         // Policy 2: AS_PATH prepending for traffic engineering

146.         // (Not done on import, but shown for completeness)

147.

148.         // Policy 3: MED handling (only compare if from same neighbor AS)

149.         // MED is already in the route

150.     }

151.

152.     /**

153.      * Core BGP best path selection algorithm

154.      * Implements the standard BGP decision process

155.      */

156.     BGPRoute SelectBestPath(const std::vector<BGPRoute>& routes) {

157.         if (routes.empty()) {

158.             return BGPRoute();

159.         }

160.

161.         if (routes.size() == 1) {

162.             return routes[0];

163.         }

164.

165.         std::vector<BGPRoute> candidates = routes;

166.

167.         // STEP 1: Prefer highest LOCAL_PREF

168.         uint32_t maxLocalPref = 0;

169.         for (const auto& r : candidates) {

170.             if (r.localPref > maxLocalPref) {

171.                 maxLocalPref = r.localPref;

172.             }

173.         }

174.

175.         candidates.erase(

176.             std::remove_if(candidates.begin(), candidates.end(),

177.                 [maxLocalPref](const BGPRoute& r) {

178.                     return r.localPref < maxLocalPref;

179.                 }),

180.             candidates.end()

181.         );

182.

183.         if (candidates.size() == 1) {

184.             NS_LOG_DEBUG("Best path selected by LOCAL_PREF: " << maxLocalPref);

185.             return candidates[0];

186.         }

187.

188.         // STEP 2: Prefer shortest AS_PATH

189.         uint32_t minASPathLen = UINT32_MAX;

190.         for (const auto& r : candidates) {

191.             if (r.GetASPathLength() < minASPathLen) {

192.                 minASPathLen = r.GetASPathLength();

193.             }

194.         }

195.

196.         candidates.erase(

197.             std::remove_if(candidates.begin(), candidates.end(),

198.                 [minASPathLen](const BGPRoute& r) {

199.                     return r.GetASPathLength() > minASPathLen;

200.                 }),

201.             candidates.end()

202.         );

203.

204.         if (candidates.size() == 1) {

205.             NS_LOG_DEBUG("Best path selected by AS_PATH length: " << minASPathLen);

206.             return candidates[0];

207.         }

208.

209.         // STEP 3: Prefer lowest ORIGIN (IGP < EGP < INCOMPLETE)

210.         uint32_t minOrigin = UINT32_MAX;

211.         for (const auto& r : candidates) {

212.             if (r.origin < minOrigin) {

213.                 minOrigin = r.origin;

214.             }

215.         }

216.

217.         candidates.erase(

218.             std::remove_if(candidates.begin(), candidates.end(),

219.                 [minOrigin](const BGPRoute& r) {

220.                     return r.origin > minOrigin;

221.                 }),

222.             candidates.end()

223.         );

224.

225.         if (candidates.size() == 1) {

226.             NS_LOG_DEBUG("Best path selected by ORIGIN: " << minOrigin);

227.             return candidates[0];

228.         }

229.

230.         // STEP 4: Prefer lowest MED (only if from same neighboring AS)

231.         // Group by neighboring AS

232.         std::map<uint32_t, std::vector<BGPRoute>> byNeighborAS;

233.         for (const auto& r : candidates) {

234.             byNeighborAS[r.receivedFromAS].push_back(r);

235.         }

236.

237.         // If all from same AS, compare MED

238.         if (byNeighborAS.size() == 1) {

239.             uint32_t minMED = UINT32_MAX;

240.             for (const auto& r : candidates) {

241.                 if (r.multiExitDisc < minMED) {

242.                     minMED = r.multiExitDisc;

243.                 }

244.             }

245.

246.             candidates.erase(

247.                 std::remove_if(candidates.begin(), candidates.end(),

248.                     [minMED](const BGPRoute& r) {

249.                         return r.multiExitDisc > minMED;

250.                     }),

251.                 candidates.end()

252.             );

253.

254.             if (candidates.size() == 1) {

255.                 NS_LOG_DEBUG("Best path selected by MED: " << minMED);

256.                 return candidates[0];

257.             }

258.         }

259.

260.         // STEP 5: Prefer eBGP over iBGP (simplified - check if neighbor AS != local AS)

261.         std::vector<BGPRoute> ebgpRoutes;

262.         for (const auto& r : candidates) {

263.             if (r.receivedFromAS != localAS) {

264.                 ebgpRoutes.push_back(r);

265.             }

266.         }

267.

268.         if (!ebgpRoutes.empty()) {

269.             candidates = ebgpRoutes;

270.         }

271.

272.         if (candidates.size() == 1) {

273.             NS_LOG_DEBUG("Best path selected by eBGP preference");

274.             return candidates[0];

275.         }

276.

277.         // STEP 6: Prefer lowest IGP cost to NEXT_HOP

278.         // (Would require IGP routing table lookup - simplified here)

279.

280.         // STEP 7: Prefer oldest route (route stability)

281.         BGPRoute oldest = candidates[0];

282.         for (const auto& r : candidates) {

283.             if (r.timestamp < oldest.timestamp) {

284.                 oldest = r;

285.             }

286.         }

287.

288.         if (oldest.timestamp < candidates[0].timestamp) {

289.             NS_LOG_DEBUG("Best path selected by age (oldest)");

290.             return oldest;

291.         }

292.

293.         // STEP 8: Prefer lowest neighbor Router ID (tie-breaker)

294.         // Use receivedFromAS as proxy for router ID

295.         BGPRoute lowestAS = candidates[0];

296.         for (const auto& r : candidates) {

297.             if (r.receivedFromAS < lowestAS.receivedFromAS) {

298.                 lowestAS = r;

299.             }

300.         }

301.

302.         NS_LOG_DEBUG("Best path selected by Router ID (AS number): "

303.                     << lowestAS.receivedFromAS);

304.         return lowestAS;

305.     }

306.

307.     /**

308.      * Helper functions

309.      */

310.     bool IsValidPrefix(const BGPRoute::NetworkPrefix& prefix) {

311.         // Check for bogon prefixes (simplified)

312.         std::string addr = prefix.network.Print();

313.

314.         // Reject 0.0.0.0/x

315.         if (addr.substr(0, 3) == "0.0") return false;

316.

317.         // Reject loopback 127.0.0.0/8

318.         if (addr.substr(0, 3) == "127") return false;

319.

320.         // Reject link-local, etc.

321.         // (More checks would be added in real implementation)

322.

323.         return true;

324.     }

325.

326.     bool RoutesEqual(const BGPRoute& r1, const BGPRoute& r2) {

327.         return r1.asPath == r2.asPath &&

328.                r1.nextHop == r2.nextHop &&

329.                r1.receivedFromAS == r2.receivedFromAS;

330.     }

331.

332.     bool IsCustomer(uint32_t asNumber) {

333.         // Check if AS is a customer (implementation-specific)

334.         return false; // Placeholder

335.     }

336.

337.     bool IsPeer(uint32_t asNumber) {

338.         // Check if AS is a peer

339.         return true; // Placeholder

340.     }

341.

342.     bool IsProvider(uint32_t asNumber) {

343.         // Check if AS is a provider

344.         return false; // Placeholder

345.     }

346.

347.     void InstallRoute(const BGPRoute& route) {

348.         NS_LOG_FUNCTION(this << route.prefix.ToString());

349.

350.         // Install route in the IP forwarding table

351.         Ptr<Node> node = /* get local node */;

352.         Ptr<Ipv4StaticRouting> staticRouting = /* get routing protocol */;

353.

354.         // Add route to forwarding table

355.         // staticRouting->AddNetworkRouteTo(

356.         //     route.prefix.network,

357.         //     route.prefix.mask,

358.         //     route.nextHop,

359.         //     interfaceIndex

360.         // );

361.     }

362.

363.     void WithdrawRoute(const BGPRoute::NetworkPrefix& prefix) {

364.         NS_LOG_FUNCTION(this << prefix.ToString());

365.         // Remove route from forwarding table

366.     }

367.

368.     void AdvertiseRoute(const BGPRoute& route) {

369.         NS_LOG_FUNCTION(this << route.prefix.ToString());

370.

371.         // Prepare route for advertisement (prepend our AS)

372.         BGPRoute advertisement = route;

373.         advertisement.PrependAS(localAS);

374.

375.         // Send to appropriate neighbors based on export policy

376.         // (Implementation would iterate through BGP neighbors)

377.     }

378.

379.     void AdvertiseWithdrawal(const BGPRoute::NetworkPrefix& prefix) {

380.         NS_LOG_FUNCTION(this << prefix.ToString());

381.         // Send withdrawal message to neighbors

382.     }

383. };

384.

385. } // namespace ns3

386.

The solution provides a comprehensive framework for modeling BGP and inter-AS routing in NS-3:

Key Design Principles:

- AS Encapsulation: Use custom container classes to logically group nodes and maintain AS boundaries

- Routing Isolation: Confine IGP protocols to internal interfaces by checking AS membership

- IXP Modeling: Create explicit peering links with proper IP addressing and interface tagging

- BGP Attributes: Full data structure supporting AS_PATH, LOCAL_PREF, MED, NEXT_HOP, and ORIGIN

- Decision Process: Standards-compliant implementation of the BGP best path algorithm

Practical Implementation Steps:

To implement this in your NS-3 simulation:

- Create AutonomousSystem objects for AS65001 (GlobalISP) and AS65002 (TransitProvider)

- Use IXPHelper::CreatePeeringPoint() to establish connections at IXP-A and IXP-B

- Configure OSPF within each AS using IntraASRoutingHelper

- Implement BGPDecisionProcess on border routers

- Simulate route announcements between ASes using the BGPRoute data structure

The BGP decision process correctly implements the standard 8-step algorithm, ensuring that routes are selected based on policy (LOCAL_PREF), path length (AS_PATH), and other tie-breaking criteria in the proper order.

C

1. // Complete BGP Decision Process Algorithm (Pseudocode)

2. class BGPDecisionProcess {

3. public:

4.     struct BGPUpdate {

5.         Ipv4Prefix prefix;

6.         Ipv4Address nextHop;

7.         vector<uint32_t> asPath;

8.         uint32_t localPref;

9.         uint32_t med;

10.         uint32_t origin; // 0=IGP, 1=EGP, 2=INCOMPLETE

11.         Time timestamp;

12.     };

13.

14.     // Main decision function

15.     BGPUpdate ProcessRouteUpdate(BGPUpdate newRoute,

16.                                 vector<BGPUpdate> existingRoutes,

17.                                 uint32_t localASN) {

18.

19.         // Step 0: Basic validation

20.         if (!ValidateRoute(newRoute, localASN)) {

21.             return BGPUpdate(); // Invalid route

22.         }

23.

24.         // Step 1: Import policies (set LOCAL_PREF based on neighbor)

25.         ApplyImportPolicy(newRoute);

26.

27.         // Step 2: Add route to RIB (Routing Information Base)

28.         vector<BGPUpdate> candidateRoutes = existingRoutes;

29.         candidateRoutes.push_back(newRoute);

30.

31.         // Step 3: Run BGP decision process

32.         BGPUpdate bestRoute = SelectBestPath(candidateRoutes);

33.

34.         // Step 4: If new route becomes best, update forwarding

35.         if (bestRoute.prefix == newRoute.prefix &&

36.             bestRoute.nextHop == newRoute.nextHop) {

37.             UpdateForwardingTable(bestRoute);

38.

39.             // Step 5: Export to other neighbors

40.             for (auto& neighbor : GetAllBGPNeighbors()) {

41.                 if (neighbor != newRoute.sender) {

42.                     BGPUpdate exportRoute = ApplyExportPolicy(bestRoute, neighbor);

43.                     SendUpdateToNeighbor(neighbor, exportRoute);

44.                 }

45.             }

46.         }

47.

48.         return bestRoute;

49.     }

50.

51. private:

52.     bool ValidateRoute(BGPUpdate& route, uint32_t localASN) {

53.         // Rule 1: Next hop must be reachable

54.         if (!IsReachable(route.nextHop)) {

55.             return false;

56.         }

57.

58.         // Rule 2: Check for AS_PATH loops

59.         if (ContainsAS(route.asPath, localASN)) {

60.             return false; // Loop detected!

61.         }

62.

63.         // Rule 3: Maximum AS_PATH length (prevent amplification)

64.         if (route.asPath.size() > MAX_AS_PATH_LENGTH) {

65.             return false;

66.         }

67.

68.         return true;

69.     }

70.

71.     BGPUpdate SelectBestPath(vector<BGPUpdate>& routes) {

72.         if (routes.empty()) return BGPUpdate();

73.

74.         BGPUpdate best = routes[0];

75.

76.         for (const auto& route : routes) {

77.             if (IsRouteBetter(route, best)) {

78.                 best = route;

79.             }

80.         }

81.

82.         return best;

83.     }

84.

85.     bool IsRouteBetter(const BGPUpdate& route1, const BGPUpdate& route2) {

86.         // BGP Decision Process Steps:

87.

88.         // 1. Highest LOCAL_PREF

89.         if (route1.localPref != route2.localPref) {

90.             return route1.localPref > route2.localPref;

91.         }

92.

93.         // 2. Shortest AS_PATH

94.         if (route1.asPath.size() != route2.asPath.size()) {

95.             return route1.asPath.size() < route2.asPath.size();

96.         }

97.

98.         // 3. Lowest ORIGIN type (IGP < EGP < INCOMPLETE)

99.         if (route1.origin != route2.origin) {

100.             return route1.origin < route2.origin;

101.         }

102.

103.         // 4. Lowest MED (Multi-Exit Discriminator)

104.         if (route1.med != route2.med) {

105.             return route1.med < route2.med;

106.         }

107.

108.         // 5. Prefer eBGP over iBGP

109.         bool route1_isEBGP = IsEBGP(route1);

110.         bool route2_isEBGP = IsEBGP(route2);

111.         if (route1_isEBGP != route2_isEBGP) {

112.             return route1_isEBGP; // eBGP preferred

113.         }

114.

115.         // 6. Lowest IGP metric to next hop

116.         uint32_t metric1 = GetIGPMetric(route1.nextHop);

117.         uint32_t metric2 = GetIGPMetric(route2.nextHop);

118.         if (metric1 != metric2) {

119.             return metric1 < metric2;

120.         }

121.

122.         // 7. Tie-breakers (simplified)

123.         // In reality: router ID, cluster list, etc.

124.         return route1.timestamp < route2.timestamp; // Older route

125.     }

126.

127.     void UpdateForwardingTable(const BGPUpdate& route) {

128.         // Pseudo-code for updating IP forwarding

129.

130.         // 1. Find interface for next hop

131.         Interface outInterface = FindInterfaceForNextHop(route.nextHop);

132.

133.         // 2. Check if route already exists

134.         ForwardingEntry existingEntry =

135.             GetForwardingEntry(route.prefix.network, route.prefix.mask);

136.

137.         if (existingEntry.exists) {

138.             // 3. Compare with existing route

139.             if (ShouldReplace(existingEntry, route)) {

140.                 // 4. Remove old entry

141.                 RemoveForwardingEntry(existingEntry);

142.

143.                 // 5. Add new entry

144.                 AddForwardingEntry(

145.                     route.prefix.network,

146.                     route.prefix.mask,

147.                     route.nextHop,

148.                     outInterface.index

149.                 );

150.

151.                 cout << "BGP: Updated forwarding for "

152.                      << route.prefix.ToString()

153.                      << " via " << route.nextHop << endl;

154.             }

155.         } else {

156.             // New route, just add it

157.             AddForwardingEntry(

158.                 route.prefix.network,

159.                 route.prefix.mask,

160.                 route.nextHop,

161.                 outInterface.index

162.             );

163.

164.             cout << "BGP: Installed new route for "

165.                  << route.prefix.ToString()

166.                  << " via " << route.nextHop << endl;

167.         }

168.

169.         // 6. Trigger route redistribution to IGP if needed

170.         if (ShouldRedistributeToIGP(route)) {

171.             RedistributeBGPtoIGP(route);

172.         }

173.     }

174. };

175.

Question 4 : Simulating a Route Leak

1. // Route Leak Simulation

2. class RouteLeakScenario {

3. public:

4.     void SimulateRouteLeak() {

5.         cout << "\n=== Simulating BGP Route Leak ===" << endl;

6.

7.         // Normal scenario: AS65001 advertises its prefix

8.         BGPRouteAttributes legitimateRoute(

9.             Ipv4Address("10.1.0.0"),

10.             Ipv4Mask("255.255.0.0"),

11.             100, // LOCAL_PREF

12.             0    // MED

13.         );

14.         legitimateRoute.AddASPath(65001);

15.         legitimateRoute.SetNextHop("100.64.1.1"); // AS65001_BR1

16.

17.         // Malicious router in AS65002 (Rogue_AS65002)

18.         // Incorrectly re-advertises AS65001's route back to AS65001

19.         BGPRouteAttributes leakedRoute(

20.             Ipv4Address("10.1.0.0"),

21.             Ipv4Mask("255.255.0.0"),

22.             150, // Higher LOCAL_PREF to attract traffic

23.             0    // MED

24.         );

25.

26.         // Malicious AS_PATH: Pretends the route came from somewhere else

27.         // AS_PATH: 65002 65001 (implies: route originated at AS65001, passed through AS65002)

28.         // But actually: route originated at AS65001, should not go through AS65002 back to AS65001

29.         leakedRoute.AddASPath(65001);

30.         leakedRoute.AddASPath(65002); // This is the leak!

31.         leakedRoute.SetNextHop("100.64.1.2"); // Rogue router in AS65002

32.

33.         cout << "Legitimate route AS_PATH: [65001]" << endl;

34.         cout << "Leaked route AS_PATH: [65002 65001]" << endl;

35.

36.         // How nodes in AS65001 react:

37.         vector<BGPRouteAttributes> candidateRoutes;

38.         candidateRoutes.push_back(legitimateRoute);

39.         candidateRoutes.push_back(leakedRoute);

40.

41.         // Run BGP decision in AS65001 router

42.         BGPRouteAttributes chosenRoute =

43.             BGPDecisionProcess::SelectBestRoute(candidateRoutes);

44.

45.         cout << "\nAS65001 Router Decision:" << endl;

46.         cout << "Option 1: " << legitimateRoute.ToString() << endl;

47.         cout << "Option 2: " << leakedRoute.ToString() << endl;

48.

49.         if (chosenRoute.GetASPath().size() > 0 &&

50.             chosenRoute.GetASPath()[0] == 65002) {

51.             cout << "RESULT: CHOOSES LEAKED ROUTE!" << endl;

52.             cout << "WHY: Higher LOCAL_PREF (150 vs 100)" << endl;

53.             cout << "CONSEQUENCE: Traffic from AS65001 to 10.1.0.0/16" << endl;

54.             cout << "  will be sent to AS65002 instead of staying internal!" << endl;

55.             cout << "  This creates a routing loop or traffic interception." << endl;

56.         } else {

57.             cout << "RESULT: CHOOSES LEGITIMATE ROUTE" << endl;

58.             cout << "WHY: Loop prevention or other attributes better" << endl;

59.         }

60.

61.         // Defensive mechanisms simulation

62.         SimulateDefenses(legitimateRoute, leakedRoute);

63.     }

64.

65. private:

66.     void SimulateDefenses(const BGPRouteAttributes& legitimate,

67.                          const BGPRouteAttributes& leaked) {

68.         cout << "\n=== Defensive Mechanisms ===" << endl;

69.

70.         // Defense 1: AS_PATH loop detection

71.         cout << "1. AS_PATH Loop Detection:" << endl;

72.         if (leaked.ContainsAS(65001)) {

73.             cout << "   LEAK DETECTED: AS_PATH contains own AS (65001)" << endl;

74.             cout << "   Action: Reject leaked route" << endl;

75.         }

76.

77.         // Defense 2: Maximum Prefix Length

78.         cout << "\n2. Maximum Prefix Advertisement:" << endl;

79.         cout << "   AS65001 could configure: 'neighbor AS65002 maximum-prefix 10'" << endl;

80.         cout << "   If AS65002 advertises >10 prefixes, shutdown session" << endl;

81.

82.         // Defense 3: Route Filtering with ROAs (RPKI)

83.         cout << "\n3. RPKI Validation:" << endl;

84.         cout << "   Check if AS65002 is authorized to advertise 10.1.0.0/16" << endl;

85.         cout << "   ROA would say: 10.1.0.0/16, origin AS65001, max length /16" << endl;

86.         cout << "   Leaked route: origin AS65002 -> INVALID" << endl;

87.

88.         // Defense 4: BGP Communities

89.         cout << "\n4. BGP Communities:" << endl;

90.         cout << "   AS65001 tags internal routes with: 65001:100 (no-export)" << endl;

91.         cout << "   AS65002 should not re-advertise these routes" << endl;

92.         cout << "   But malicious router ignores communities" << endl;

93.     }

94. };

95.

96. // Example of malicious advertisement

97. void CreateMaliciousRouteAdvertisement() {

98.     cout << "\n=== Malicious Router in AS65002 ===" << endl;

99.     cout << "Normal behavior:" << endl;

100.     cout << "  Receive: 10.1.0.0/16 from AS65001 (AS_PATH: [65001])" << endl;

101.     cout << "  Export to: Other customers (AS_PATH: [65002 65001])" << endl;

102.     cout << "  Should NOT export to: AS65001 (would create [65002 65001])" << endl;

103.

104.     cout << "\nMalicious behavior:" << endl;

105.     cout << "  Receive: 10.1.0.0/16 from AS65001" << endl;

106.     cout << "  Modify: Set LOCAL_PREF = 150" << endl;

107.     cout << "  Re-advertise to: AS65001 (violating policy)" << endl;

108.     cout << "  Resulting AS_PATH: [65002 65001]" << endl;

109.

110.     cout << "\nImpact:" << endl;

111.     cout << "1. AS65001 sees two paths to its own network:" << endl;

112.     cout << "   Path A: Direct (AS_PATH: [], cost 0)" << endl;

113.     cout << "   Path B: Via AS65002 (AS_PATH: [65002], LOCAL_PREF 150)" << endl;

114.     cout << "2. BGP selects Path B due to higher LOCAL_PREF" << endl;

115.     cout << "3. Traffic loops: AS65001 -> AS65002 -> AS65001" << endl;

116.     cout << "4. AS65002 can intercept, inspect, or drop traffic" << endl;

117. }

118.

119.

Question 5

1. // Discussion of NS-3 BGP Model vs Reality

2. class BGPModelLimitations {

3. public:

4.     struct RealBGPFeature {

5.         string name;

6.         string description;

7.         string whyHardToModel;

8.     };

9.

10.     void CompareWithReality() {

11.         cout << "\n=== NS-3 BGP Model vs Real BGP ===" << endl;

12.

13.         cout << "\nSIGNIFICANT SIMPLIFICATIONS:" << endl;

14.         cout << "1. No TCP Session Management:" << endl;

15.         cout << "   Real BGP: Runs over TCP (port 179), with session establishment," << endl;

16.         cout << "             keepalives, graceful restart" << endl;

17.         cout << "   NS-3: Direct message passing, no transport layer simulation" << endl;

18.

19.         cout << "\n2. Simplified State Machine:" << endl;

20.         cout << "   Real BGP: 6-state FSM (Idle, Connect, Active, OpenSent," << endl;

21.         cout << "             OpenConfirm, Established)" << endl;

22.         cout << "   NS-3: Simple 'connected/disconnected' state" << endl;

23.

24.         cout << "\n3. No Route Dampening:" << endl;

25.         cout << "   Real BGP: Penalizes flapping routes, suppresses unstable prefixes" << endl;

26.         cout << "   NS-3: All routes treated equally, no stability tracking" << endl;

27.

28.         cout << "\n4. No Real-world Policies:" << endl;

29.         cout << "   Real BGP: Complex regular expressions for AS_PATH filtering," << endl;

30.         cout << "             prefix-lists, route-maps with set/delete community" << endl;

31.         cout << "   NS-3: Simple if-else statements for policy" << endl;

32.

33.         cout << "\n5. Scale Limitations:" << endl;

34.         cout << "   Real Internet: ~900,000 BGP routes in global table" << endl;

35.         cout << "   NS-3: Typically <1000 routes due to memory/performance" << endl;

36.

37.         // Three Critical Features Extremely Difficult to Model

38.         vector<RealBGPFeature> difficultFeatures = {

39.             {

40.                 "BGP Route Reflectors",

41.                 "Hierarchical route distribution within an AS to reduce iBGP mesh",

42.                 "Requires modeling complex cluster relationships, client/non-client roles, "

43.                 "originator ID, cluster list attributes. NS-3's flat node hierarchy "

44.                 "doesn't naturally support the administrative relationships needed."

45.             },

46.             {

47.                 "BGP Large Communities & Extended Communities",

48.                 "Complex policy tagging with 12-byte communities for traffic engineering",

49.                 "NS-3's packet headers are simplified. Modeling the full 12-byte community "

50.                 "attribute with transitive/non-transitive flags and complex matching "

51.                 "semantics would require extensive modifications to packet structure "

52.                 "and attribute propagation logic."

53.             },

54.             {

55.                 "gRPC/Protobuf-based BGP (OpenConfig)",

56.                 "Modern BGP monitoring and configuration via gRPC with YANG models",

57.                 "NS-3 focuses on network layer simulation. Implementing full gRPC stack "

58.                 "with Protobuf serialization, YANG model validation, and NETCONF/RESTCONF "

59.                 "interfaces would be equivalent to building a real network operating "

60.                 "system, not a simulation."

61.             }

62.         };

63.

64.         cout << "\nTHREE CRITICALLY DIFFICULT FEATURES TO MODEL:" << endl;

65.         for (size_t i = 0; i < difficultFeatures.size(); i++) {

66.             cout << "\n" << (i+1) << ". " << difficultFeatures[i].name << ":" << endl;

67.             cout << "   What: " << difficultFeatures[i].description << endl;

68.             cout << "   Why hard: " << difficultFeatures[i].whyHardToModel << endl;

69.         }

70.

71.         // Assessment of NS-3 for Inter-AS Research

72.         cout << "\n=== IS NS-3 SUITABLE FOR INTER-AS ROUTING RESEARCH? ===" << endl;

73.

74.         cout << "\nYES, for certain types of research:" << endl;

75.         cout << "1. Protocol Behavior Studies:" << endl;

76.         cout << "   - BGP convergence after link failures" << endl;

77.         cout << "   - Route propagation dynamics" << endl;

78.         cout << "   - Impact of BGP timers (MRAI)" << endl;

79.

80.         cout << "\n2. Security Research:" << endl;

81.         cout << "   - Route leak/prefix hijack impact analysis" << endl;

82.         cout << "   - RPKI/ROA validation effectiveness" << endl;

83.         cout << "   - BGPsec simulation (cryptographic validation)" << endl;

84.

85.         cout << "\n3. Educational Purposes:" << endl;

86.         cout << "   - Teaching BGP fundamentals" << endl;

87.         cout << "   - Visualizing AS_PATH selection" << endl;

88.         cout << "   - Understanding iBGP vs eBGP" << endl;

89.

90.         cout << "\nNO, for production or highly realistic studies:" << endl;

91.         cout << "1. Performance Testing:" << endl;

92.         cout << "   - Cannot simulate full Internet routing table" << endl;

93.         cout << "   - No hardware acceleration or TCAM simulation" << endl;

94.

95.         cout << "\n2. Interoperability Testing:" << endl;

96.         cout << "   - Cannot test against real router implementations" << endl;

97.         cout << "   - No vendor-specific feature simulation" << endl;

98.

99.         cout << "\n3. Microsecond-level Timing:" << endl;

100.         cout << "   - NS-3 abstracts lower-layer timing" << endl;

101.         cout << "   - No accurate simulation of BGP UPDATE packet processing delays" << endl;

102.

103.         cout << "\nRECOMMENDED APPROACH:" << endl;

104.         cout << "Hybrid methodology:" << endl;

105.         cout << "1. Use NS-3 for large-scale topology and protocol behavior" << endl;

106.         cout << "2. Use MiniNet or container-based emulation for real BGP daemons" << endl;

107.         cout << "3. Use historical BGP data (RouteViews, RIPE RIS) for validation" << endl;

108.         cout << "4. For production: Test on physical/virtual routers (FRR, Bird)" << endl;

109.

110.         cout << "\nCONCLUSION:" << endl;

111.         cout << "NS-3 is EXCELLENT for:" << endl;

112.         cout << "- Understanding BGP algorithmic behavior" << endl;

113.         cout << "- Educational visualization" << endl;

114.         cout << "- Controlled experiments with known variables" << endl;

115.         cout << "- Research on new BGP extensions in idealized conditions" << endl;

116.

117.         cout << "\nNS-3 is INADEQUATE for:" << endl;

118.         cout << "- Testing real-world BGP implementations" << endl;

119.         cout << "- Performance benchmarking" << endl;

120.         cout << "- Interoperability validation" << endl;

121.         cout << "- Production network design validation" << endl;

122.

123.         cout << "\nUse NS-3 as a complement to, not replacement for, real BGP testing." << endl;

124.     }

125. };

126.

127. // Hybrid approach suggestion

128. void HybridResearchMethodology() {

129.     cout << "\n=== Recommended Research Pipeline ===" << endl;

130.     cout << "Phase 1: NS-3 Simulation" << endl;

131.     cout << "  - Model new BGP extension algorithmically" << endl;

132.     cout << "  - Test in controlled, repeatable environment" << endl;

133.     cout << "  - Validate basic correctness" << endl;

134.

135.     cout << "\nPhase 2: Emulation (MiniNet, GNS3)" << endl;

136.     cout << "  - Implement in real BGP daemon (FRRouting)" << endl;

137.     cout << "  - Test interoperability with existing implementations" << endl;

138.     cout << "  - Measure real performance metrics" << endl;

139.

140.     cout << "\nPhase 3: Testbed Deployment" << endl;

141.     cout << "  - Deploy on research testbed (GENI, FABRIC)" << endl;

142.     cout << "  - Test at scale with realistic traffic" << endl;

143.     cout << "  - Collect operational experience" << endl;

144.

145.     cout << "\nPhase 4: Real-world Deployment" << endl;

146.     cout << "  - Deploy in controlled production environment" << endl;

147.     cout << "  - Monitor with real BGP looking glasses" << endl;

148.     cout << "  - Collect BGP update data for analysis" << endl;

149. }

150.

Summary of Key Points:

- Route Leak Behavior: Malicious router creates AS_PATH [65002 65001] which should be rejected by AS65001 due to loop detection (contains own AS). However, if LOCAL_PREF is manipulated higher, it might be temporarily preferred until loop detection kicks in.

- NS-3 Limitations: The model simplifies TCP, state machines, route dampening, and lacks complex policies. Critical features like route reflectors, communities, and gRPC are extremely difficult to model accurately.

- Suitability Assessment: NS-3 is suitable for algorithmic studies, educational purposes, and controlled experiments but inadequate for performance testing, interoperability validation, or production network design.

The most valuable use of NS-3 for BGP research is in understanding fundamental behaviors and testing new algorithms in a controlled environment before moving to more realistic emulation or testbed deployment.